# Facilities Planning Model Assessment of

# Sports Hall Provision for

# Greater London Authority

# Standard Report

# May 2026

This document has been produced by Sport England for the Greater London Authority in March 2026. This document can be reproduced by the Greater London Authority, subject to it being used accurately and not in a misleading context.  When the document is reproduced in whole or in part within another publication or service, the full title, date, and accreditation to Sport England must be included.

Disclaimer

The information in this report is presented in good faith using the information available to Sport England at the time of preparation.  It is provided on the basis that the authors of the report are not liable to any person or organisation for any damage or loss which may occur in relation to taking, or not taking, action in respect of any information or advice within the document.

Sport England assumes no responsibility for the completeness, accuracy and currency of the information contained on the maps taken from the Active Places Power website and its terms and conditions apply.

Accreditations

Other than data provided by Sport England, this report also contains data from the following sources:

Ordnance Survey data © Crown copyright and database rights 2025 OS AC0000850566.

National Statistics data © Crown copyright and database right 2025.

Population distribution based on 2021 Census data for output areas.  Population for 2025 modified by 2018-based Subnational Population Projections for Local Authorities. Adapted from data from the Office for National Statistics licensed under the Open Government Licence v.3.0. London boroughs modified by © GLA 2022-based demographic projections - ward populations, identified capacity scenario.

Index of Multiple Deprivation data contains public sector information licensed under the Open Government Licence v3.0.

Executive Summary

This report provides an initial assessment of the current supply and demand provision for sports halls across London in 2025. It has been prepared based on an assessment using the Sport England Facilities Planning Model (FPM) spatial modelling tool. The report sets out the findings under eight different headings. The findings are reported by way of a series of tables and maps supported by some analysis. The report has been written, focusing on a London wide picture of provision. There will be supply and demand differences across the London boroughs, some of which will be drawn out in this report. The report should form part of a wider assessment of provision at the local level, which then provides a rounded assessment and evidence base report.   For a detailed assessment at a borough wide level, it is recommended that, where necessary, each individual borough undertakes a strategic planning exercise to fully understand its current supply and demand position.

Inclusion Criteria

The FPM spatial modelling tool excludes facilities that are deemed to be either for private use, too small, closed or there is a lack of information, particularly relating to hours of use. For more information about excluded facilities please see Appendix 3 (Facility Inclusion Criteria).

Important Definitions

- Satisfied Demand – This represents the proportion of total demand that is met by the capacity at the sports halls for London residents who live within the driving, walking or public transport travel time of a sports hall. This includes sports halls located both within and outside London.

- Unmet Demand – Demand for sports halls that cannot be met because either:

  - There is too much demand for any particular sports hall within its travel time area and there is a lack of capacity.

  - The demand is located too far from any sports halls that it can use (taking into account deprivation) or reach and is then classified as unmet demand.

- Used Capacity – A measure of usage at sports halls and an estimate for how well used or full facilities are. The FPM is designed to include a ‘comfort factor’, beyond which the venues are too full. In the model Sport England assumes that usage of more than 80% of capacity is busy and that the sports hall is operating at an uncomfortable level above that percentage.

- Badminton Courts – badminton courts are used as a standard unit of measurement to calculate the supply and demand of a sports hall. Further information on the use of badminton courts as a unit of measurement to calculate capacity can be found on page 80.

Key Findings

The key findings from the supply, demand and access assessment are as follows:

There are a total of 684 sports halls on 481 sites in London that equates to 2,616 badminton courts.  The space available for community use in the weekly peak period is the equivalent of 1,859 courts therefore the unavailable court space equates to 28.9% of the total supply of badminton courts. This is because much of London’s sports hall provision is based on education sites and is inaccessible at peak times.

Bromley is the borough with the highest number of sports halls and Hammersmith and Fulham has the fewest. The City of London has been included in the report but has no sports halls and therefore will not be included in the analysis. The average age for public sports halls in London is 33 years old. This is quite old and impacts on the quality of users’ experience.

The sports halls can accommodate a total of 684,075 visits in the weekly peak period however the resident population generates demand for 709,045 visits in the weekly peak period, which equates to 2,408 courts with a comfort factor included.  This indicates that not all peak period demand can currently be accommodated.

Of the demand for sports halls from London’s residents, 90% is met in 2025.  This percentage is below the national average of 95%.  Sutton has the highest demand met (98%) while Hammersmith and Fulham (70%) has the lowest demand met.

Circa 96% of London’s satisfied demand is met within London.  A subset of the satisfied demand findings shows how much of London’s residents’ demand for sports halls is met at sports halls located within each borough. Hammersmith & Fulham (70%) retain the least satisfied demand of all the Boroughs whereas Newham retains the most (86%).

Unmet demand for sports halls totals 239 courts. The locations with the highest amounts of unmet demand in 2025 are located in the London Boroughs of Hammersmith and Fulham, on the border between Westminster and Camden, Southwark and Tower Hamlets (see dark red squares on Map 6.1). In general, the areas of greatest unmet need are in the inner London boroughs, with the outer London Boroughs having very low levels of unmet need.

The overall estimated used capacity of sports halls in London during the weekly peak period is 92%.  Only the London Borough of Havering has a used capacity lower than Sport England’s comfort level of 80% of capacity used at peak times with Bromley, Kingston upon Thames and Sutton meeting this comfort level.

The model estimates that there are 10 Boroughs where the estimated used capacity of the sports halls is 100%. In these authorities a reduction in used capacity cannot be created unless there is increased access to the existing sports halls, where there is limited access, or more sports halls are provided.

Local share in London is 0.5 meaning that there is insufficient suitable provision that the demand can reach.  While there is some variation in the geographical distribution of local share across London, local share is still below 1 in all Boroughs. Broadly, Local share is worse in the inner Boroughs and comparatively better in the outer Boroughs. There is no London Borough where local share meets or exceeds the average share across England (see table 8.2).

Strategic Overview

There is an under-supply of available sports halls that can be accessed by London’s residents.  This equates to 239 courts of unmet demand (with the 80% comfort factor applied). Furthermore, the utilisation of all the available halls is on average above the comfortable capacity level of 80%.  Increasing access to education sports halls would reduce considerably the London wide average for used capacity of sports halls and would create a more even and lower distribution of demand. However, if increased access to education sites was achieved, there would still be a need to increase the provision of sports halls as there is a lack of capacity overall and education halls are not available during core school hours consequently cannot be relied on to address all identified deficits.

The current supply of sports halls needs to be protected and enhanced.  If Community Use Agreements (CUAs) are not already in place, they should be negotiated and agreed.  Sport England can advise on a CUA for any new or replacement sports hall as part of the planning process.  It will be important to ensure that delivery of any CUA is monitored.

Contents

Executive Summary	3

Section 1: Introduction	7

Section 2: Sports Hall Supply	11

Section 3: Demand for Sports Halls	15

Section 4: Accessibility	20

Section 5: Satisfied Demand	25

Section 6: Unmet Demand	30

Section 7: Used Capacity	37

Section 8: Local Share of Facilities	43

Appendix 1: Sports Halls included in the assessment 2025	49

Appendix 2: Information not included within the national run	75

Appendix 3: Model Description, Inclusion Criteria and Model Parameters	78

## Section 1: Introduction

- This report which looks at all Boroughs across London provides an initial assessment of its current supply and demand for the provision of sports halls in 2025.  This assessment uses Sport England’s Facilities Planning Model (FPM) and outputs from the National Run using Active Places data as of March 2025.

- The supply assessment is based on sports halls being open and accessible for community use.  If sports halls are closed temporarily for any reason, the asset owner should inform Sport England Active Places Power via the contact us link at https://www.activeplacespower.com.

- This standard run provides an initial assessment of the current supply and demand for the provision of sports halls across London. The assessment does not include future population growth projections but is a baseline evidence base for sports hall provision.

Context

- The report should form part of a wider assessment of provision at local level, which then provides a rounded assessment and evidence base report.  This should include other available information and knowledge from:

- A sports perspective, such as national sports governing bodies and other sports organisations

- A local perspective from each London borough, the facility operators and local sports clubs

- The findings from this FPM standard report should be reviewed and applied with reference to the strategic direction being set by Sport England on:

- The policies, programmes and interventions proposed to increase sports participation and physical activity

- The application of the research applied by Sport England in determining the strategy and the evidence base

- The role that sports facilities can play in increasing sports participation and physical activity

- The strategy can be accessed at Uniting the Movement | Sport England.

- This report uses the information available on Active Places in March 2025.  In January 2026 the GLA and Sport England contacted all local authorities to understand what has changed since March 2025 and identify any significant differences in the Active Places data. This information is included in Appendix 2 and will be added to Active Places. It will be used to inform a future assessment.

  - Future Assessment

- Longer-term bespoke FPM local assessments for future provision can be undertaken based on:

- Review of these findings

- Projected population growth

- Options for changes in supply – closures/new openings at the same or different locations and on different scales

- The purpose is to identify the impact of these changes on access to sports halls for residents in future years and whether changes in supply meet future demand.

- These findings can be applied as an evidence base in regional policy, and the future assessments can also inform a long-term evidence base for securing inward investment – funding applications and new developments.

Report Structure, Content and Sequence

- Reporting the main findings follows a sequence of setting out the data in a table from the FPM analysis and then to provide a bullet point commentary on the main findings.

- Based on these findings, then specific maps or further tables/graphs are included to explain in more detail the key findings.

- This report sets out the full findings under seven assessment headings as follows:

- Supply – How many facilities are there and what is their capacity?

- Demand – Who wants to use the facilities?

- Accessibility – How do people travel to the facilities?

- Satisfied Demand – How many people use the facilities?  Where do people use the facilities (inside and outside the local authority)?

- Unmet Demand – Who is unable to use the facilities and why?  Is there insufficient capacity or are people too far away from the facilities?

- Used Capacity – How full are the facilities and where are people coming from (inside and outside London)?

- Local Share – Which areas have better or worse provision, considering the number of people who want to use them?

- Each assessment heading has a table of main findings from London followed by a full definition of the assessment. The report then looks at all London Boroughs showing the highest and lowest figures highlighted.

- Maps to support the findings on facility locations, demand, deprivation, walking coverage, unmet demand and local share are also included.

- The facilities included in the study are listed in Appendix 1.  The information collected through the Borough survey in December 2025 that is not included in the March 2025 analysis is set out in Appendix 2.  The facility planning inclusion criteria and model parameters are described in Appendix 3.

The study area

- Describing the study area provides some points of explanation and a context for the report’s findings. Customers of sports facilities do not reflect local authority boundaries and whilst there are management and pricing incentives (and possibly disincentives) for customers to use sports facilities located in the area in which they live, the reality is that people who use sports halls travel across local authority boundaries.

- Consequently, in determining the position for London it is important to take account of the sports halls in the neighbouring local authorities to London.  In particular, to assess the impact of overlapping catchment areas of facilities. Taking account of all these factors is done by establishing a study area which places London at the centre of the study and assesses the import and export of demand into and out of London and across London boroughs.

## Section 2: Sports Hall Supply

- The report on the main findings follows a sequence of setting out the data in a table for the 2025 findings from the FPM analysis. Then to provide a bullet point commentary on the main findings.

- Based on these findings, specific maps or further tables/graphs are included to explain in more detail the key findings.

- The City of London is included in the report but there are no sports halls within its area.

Table 2.1:  Supply of sports halls for London in 2025

| LONDON TOTAL |  |
| --- | --- |
| Number of halls | 684 |
| Number of hall sites | 481 |
| Supply of total hall space expressed as main court equivalents | 2,616 |
| Supply of hall space in courts, scaled by hours available in the peak period | 1,859 |
| Supply of total hall space in visits per week peak period | 684,075 |
| Average year built (all sites) | 1996 |
| Average year built (public sites) | 1993 |

Definition of supply – This is the supply or capacity of the sports halls available for community and club use in the weekly peak period.  Supply is expressed in the number of visits that a sports hall can accommodate in the weekly peak period and in badminton courts.

Weekly peak period – This is when the majority of visits take place and when users have most flexibility to visit.  The peak period hours for sports halls total 46 per week (see Appendix 2). The modelling and recommendations are based on the ability of the public to access facilities during this weekly peak period.

- The key findings for supply across London are:

- In 2025 there were 684 sports halls on 481 sites, across all the London boroughs.

- This is the total supply of sports halls and equates to 2,616 badminton courts. When the supply is assessed based on the supply available for community use in the weekly peak period (referred to as the effective supply), this reduces to 1,859 badminton courts. The reason for the difference between the total and effective supply is mainly due to the reduced hours for community use at sports halls on education sites. Each individual school, college or higher education institution will determine its own policy, type and amount of community use of sports halls.   The average age for public sports halls in London is 33 years old.  This is quite old and impacts on the quality of users’ experience.

- The difference between the total supply of sports halls and the effective supply in 2025 is 757 badminton courts or 28.9% of the total supply in badminton courts. Or put another way, 189 sports halls if the supply was in 4 badminton court size sports halls, a large difference between the total and available supply of sports halls in peak hours. Before considering the need for further provision of sports halls, assuming the demand findings indicate this is required, an initial assessment should be to consider the scope to increase access to the sports halls which have reduced or no access for community use. In effect, to make more effective use of the existing supply.

- Each of the boroughs’ supply of sports halls in 2025 is set out in in Table 2.2 below. The boroughs with the lowest supply of community accessible sports halls are highlighted in pink and the boroughs with the highest are highlighted in green. Omitting the City of London that has no sports hall, in terms of the lowest supply there is quite a gap between the lowest two supplies in Hammersmith and Fulham with 4 sports halls and Kensington and Chelsea with 6 sports halls and the next two lowest, Camden with 12 sports halls and Harrow with 15 sports halls.  The highest supply of sports halls is in Bromley with 35 sports halls, then Barnet with 34 and Newham and Redbridge that both have 32.

Table 2.2: Sport Hall Provision across all boroughs in 2025

| Borough | Number of halls | Borough | Number of halls | Borough | Number of halls |
| --- | --- | --- | --- | --- | --- |
| City of London | 0 | Merton | 17 | Hillingdon | 24 |
| Hammersmith & Fulham | 4 | Islington | 18 | Greenwich | 25 |
| Kensington & Chelsea | 6 | Hackney | 18 | Havering | 26 |
| Camden | 12 | Richmond upon Thames | 19 | Haringey | 26 |
| Harrow | 15 | Waltham Forest | 21 | Sutton | 26 |
| Barking & Dagenham | 16 | Southwark | 22 | Bexley | 29 |
| Lewisham | 16 | Brent | 22 | Croydon | 31 |
| Ealing | 16 | Hounslow | 22 | Newham | 32 |
| Westminster | 17 | Lambeth | 23 | Redbridge | 32 |
| Tower Hamlets | 17 | Enfield | 23 | Barnet | 34 |
| Kingston upon Thames | 17 | Wandsworth | 23 | Bromley | 35 |

Map 2.1: Sports Hall Site Locations in 2025

## Section 3: Demand for Sports Halls

Table 3.1: Demand for Sports halls for London in 2025

| LONDON TOTAL |  |
| --- | --- |
| Population | 9,098,148 |
| Visits demand per week in peak period | 709,045 |
| Demand in courts with comfort factor included | 2,408 |
| % of demand in the 10% most deprived LSOAs nationally | 2% |

Definition of total demand – This represents the total demand for sports halls by gender and for six age bands from 0 to 79 and is calculated as the percentage of each age band/gender that participates. This is added to the frequency of participation in each age band/gender to arrive at a total demand figure, which is expressed in visits in the weekly peak period and badminton courts.  The FPM parameters for the percentage of participation and frequency of participation, for gender and for different age bands, are calculated from Sport England’s Active Lives survey up to November 2023 and are set out in Appendix 3.

Resident Population Demand

- The Office for National Statistics 2018-based population projection for the London (GLA) region in 2025 is 9,098,148

- A key finding is that the resident population generates demand for 709,045 visits in the weekly peak period, which equates to 2,408 badminton courts with a comfort factor included. This demand is 549 courts more than are currently available for community use in the peak period meaning that 22.8% of peak period demand is not accommodated.

Geographical Distribution of Demand

- The distribution of demand for sports halls is set out in Map 3.1. The demand is expressed in scale of badminton courts with each square colour coded to represent the amount of demand in that 1-kilometre grid square. Purple squares have the lowest value with demand for less than 0.5 of one badminton court. Dark red squares represent the greatest demand at more than 5 courts.

- The area of highest demand is in one 1-kilometre square area in the Isle of Dogs, south Tower Hamlets at 6 courts. North of the river, there are also areas of highest demand in mid Tower Hamlets and central Newham with larger areas of slightly less demand across Islington, Hackney, Newham and Waltham Forest, and more dispersed areas of high demand in Westminster, Kensington and Chelsea and Hammersmith and Fulham. South of the river, the areas of highest demand are in north Southwark and to a lesser extent, in north Lambeth.

- The areas of lowest demand are in central Westminster, the City of London, and in the outer London boroughs, most notably in the outer areas of Hillingdon, Havering, Bexley, Bromley and Croydon.

Deprivation

- Map 3.2 shows areas of deprivation across London. In 2025, 2% of London’s demand for sports halls is in the 10% most deprived lower super output areas (LSOAs) nationally. However, deprivation varies across boroughs, with the highest concentrations of greatest deprivation (dark red on map 3.2) generally occurring in an arc of London north, central and east boroughs including east Enfield, east Haringey, Islington, Hackney, north Tower Hamlets, south Newham and Barking and Dagenham. There are also pockets of highest deprivation in Ealing, Brent, north Kensington and Chelsea, south Hammersmith and Fulham, south Barnet, north Westminster, Waltham Forest and Havering north of the Thames. South of the river, the areas of highest 10% deprivation are concentrated in Southwark, Lewisham and north Croydon with a small pocket in the Abbey wood area of Greenwich. Levels of deprivation are generally less (shaded green on map 3.2) towards the edge of the study area in the outer London boroughs.

- The Index of Multiple Deprivation (IMD) score is used in the FPM to limit whether people will use commercial facilities (see Appendix 3 for definition of IMD).  A weighting factor is incorporated to reflect the cost element normally associated with commercial facilities.  The assumption is that the higher the IMD score (less affluence), the less likely the population of the LSOA would choose to go to a commercial facility.

Map 3.1: Distribution of total demand for sports halls across London

FPM peak period demand per square kilometre expressed as courts (figure labels) and shown thematically (colours)

Map 3.2: Deprivation in 2025 (Index of Multiple Deprivation 2019)

Deprivation shown thematically (colours) at lower super output area level by decile.

### Section 4: Accessibility

Table 4.1: Accessibility of Halls for London in 2025

| LONDON TOTAL |  |
| --- | --- |
| % of population without access to a car | 40% |
| % of total population within a 20-minute walk of a hall | 81% |
| % of 10% most deprived population within a 20-minute walk of a hall | 88% |
| % of demand satisfied when travelled: |  |
| by car | 60% |
| on foot | 27% |
| by public transport | 13% |

Definition of accessibility – The FPM uses a distance decay function where the further a user is from a facility, the less likely they will travel.  A description of the distance decay function is set out in Appendix 2. On average, a 20-minute travel time accounts for approximately 90% of journeys to sports halls.

Car Access

- The national model includes access to sports halls by car. The percentage of the population without access to a car influences travel patterns to sports halls. A low percentage means that there is likely to be a larger number of journeys to sports halls by car.  For residents without access to a car, travel to sports halls by public transport and on foot become the choice of travel mode.

- Across London 40% of the population does not have access to a car, reflecting the greater range of transport options available in London. This is above the national average of 23%.

- It is estimated that 60% of journeys to sports halls by London residents are made by car in 2025.  This is significantly less than the national average of 74%. Rates of car use vary widely within London and will also be affected by access to public transport.

Walking Access

- Across London, visits by walking and public transport and cycling combined, represents 40% of all visits to sports halls in 2025 (27% on foot and 13% by public transport or cycling).  For context, the England wide visit rate by these modes amount to 25% (16% and 9% respectively) highlighting how Londoners disproportionately use alternative means to the car to access facilities due to proximity, alternative transport being available and less car ownership.

- 81% of London residents are within a 20-minute walk of a sports hall and this rises to 88% of residents in the 10% most deprived LSOAs.  Access is generally better, but not exclusively, in the inner London boroughs. Table 4.2 shows that rates of walking to sports halls is highest in Islington at 50% followed by Westminster, City of London, Hackney, Tower Hamlets and Camden. The fewest proportion of walking trips is in Bromley at 13% followed by Hillingdon, Harrow, Havering, Ealing and Richmond on Thames.

- Map 4.1 illustrates the extent of the walking catchment area of sports halls, (20 minutes/1mile). It illustrates an extensive area of London is inside the walking catchment area of at least one sports hall (area shaded beige). In the areas shaded orange residents have access to 2 sports halls based on the walking catchments and in the pink area it is access to 3 sports halls

- Access to sports halls is highest in the inner London area where there is the greatest concentration of the total 684 sports halls across London.

- There is a very high visit rate to sports halls by residents who walk in the City of London, Camden, Islington, Tower Hamlets and Westminster. As Map 4.1 illustrates, nearly all of the land area of these boroughs other than in the City of London, is inside the walking catchment area of at least one sports hall, so there is very high accessibility by walking.

- There are large parts of London however, which are outside the walking catchment area of a sports hall as Map 4.1 illustrates as being unshaded (white), highlighting that increasing access to sports halls based on walking catchments is important particularly for residents who don’t have access to a car.

Table 4.2: Visits to halls by walking for each London borough as a percent of satisfied demand

| Borough | % | Borough | % | Borough | % |
| --- | --- | --- | --- | --- | --- |
| Bromley | 13% | Barking & Dagenham | 22% | Waltham Forest | 32% |
| Hillingdon | 14% | Enfield | 23% | Haringey | 38% |
| Harrow | 17% | Merton | 23% | Lambeth | 39% |
| Havering | 17% | Kingston upon Thames | 23% | Southwark | 40% |
| Ealing | 17% | Redbridge | 23% | Newham | 40% |
| Hounslow | 18% | Brent | 24% | Camden | 42% |
| Croydon | 19% | Sutton | 24% | Tower Hamlets | 43% |
| Barnet | 20% | Greenwich | 25% | Hackney | 45% |
| Bexley | 20% | Lewisham | 25% | City of London | 47% |
| Richmond upon Thames | 20% | Kensington & Chelsea | 28% | Westminster | 48% |
| Hammersmith & Fulham | 22% | Wandsworth | 29% | Islington | 50% |

Map 4.1: Walking catchment area of sports halls in 2025

## Section 5: Satisfied Demand

Demand from residents currently being met by supply

Table 5.1: Satisfied Demand for Sports Halls for London in 2025

| LONDON TOTAL |  |
| --- | --- |
| Total number of visits which are met (visits per week period) | 638,582 |
| % of total demand satisfied | 90% |
| Demand of Satisfied Demand Retained (visits per week peak period) | 615,872 |
| Demand Retained - as a % of Satisfied Demand | 96% |
| Demand Exported (visits per week peak period) | 22,709 |
| Demand Exported - as a % of Satisfied Demand | 4% |

Definition of satisfied demand – This represents the proportion of total demand that is met by the capacity at the sports halls from London residents who live within the driving, walking or public transport travel time of a sports hall.  This includes sports halls located both within and outside London.

Demand Met

- The total peak period demand for visits across London per week is 709,045. The available sports halls in the weekly peak period meet 638,582 of those visits demanded which equates to 90% satisfied demand across the London area in 2025. This is significantly below the national average of 95%.

- The findings for satisfied demand for individual boroughs are set out in table 5.2 with the five highest and lowest levels of satisfied demand identified in green and pink respectively. The difference between the highest and lowest demand being satisfied is significant at 98% of total demand being met in Sutton and only 70% being met in Hammersmith and Fulham.

Table 5.2: Satisfied demand for sports halls across London boroughs

| Borough | Percentage of Satisfied Demand | Borough | Percentage of Satisfied Demand | Borough | Percentage of Satisfied Demand |
| --- | --- | --- | --- | --- | --- |
| Hammersmith & Fulham | 70% | Westminster | 88% | Barnet | 94% |
| Kensington & Chelsea | 72% | Greenwich | 89% | Waltham Forest | 94% |
| City of London | 81% | Hackney | 89% | Harrow | 94% |
| Southwark | 82% | Hounslow | 90% | Enfield | 94% |
| Lewisham | 83% | Barking & Dagenham | 90% | Richmond upon Thames | 95% |
| Tower Hamlets | 86% | Haringey | 91% | Bromley | 95% |
| Ealing | 86% | Croydon | 92% | Redbridge | 96% |
| Brent | 86% | Islington | 92% | Havering | 96% |
| Camden | 87% | Newham | 93% | Kingston upon Thames | 97% |
| Wandsworth | 87% | Merton | 93% | Bexley | 97% |
| Lambeth | 87% | Hillingdon | 93% | Sutton | 98% |

Retained Demand

Definition of retained demand – A subset of the satisfied demand findings shows how much of London’s residents’ demand for sports halls is met at sports halls located within the borough. This assessment is based on the travel time from each London borough sports hall and residents in the borough participating at these sports halls.

- It is possible to measure how much of the satisfied demand for sports halls is retained within London as a whole and also within each Borough. This is on the basis of residents using the nearest sports hall to where they live and the sports hall being in the same Borough. In 2025 retained demand within London was 96% amounting to 615,872 satisfied visits per week.

- Retained demand within each borough does vary considerably and retained demand as a % of total satisfied demand for all Boroughs in 2025 is shown in Table 5.4, with the five highest highlighted in green and the five lowest in pink.

- Excluding the City of London that doesn’t retain any of its sports hall demand, the lowest percentage of retained demand is in Hammersmith and Fulham at only 23% followed distantly by Kensington and Chealsea at 45%, Lewisham 47%, Ealing 56% and Camden 59%. The Boroughs that retain the greatest demand are Newham 86%, Haringey 84%, Westminster 83%, Waltham Forest 81% and Bexley 80%.

Table 5.4: retained demand as a % of total satisfied demand

| Borough | % | Borough | % | Borough | % |
| --- | --- | --- | --- | --- | --- |
| City of London | 0% | Barking & Dagenham | 65% | Redbridge | 73% |
| Hammersmith & Fulham | 23% | Hounslow | 65% | Barnet | 74% |
| Kensington & Chelsea | 45% | Greenwich | 68% | Kingston upon Thames | 75% |
| Lewisham | 47% | Enfield | 69% | Islington | 76% |
| Ealing | 56% | Croydon | 69% | Sutton | 78% |
| Camden | 59% | Hackney | 69% | Bromley | 79% |
| Harrow | 60% | Wandsworth | 70% | Bexley | 80% |
| Southwark | 62% | Richmond upon Thames | 70% | Waltham Forest | 81% |
| Merton | 64% | Brent | 71% | Westminster | 83% |
| Lambeth | 65% | Hillingdon | 72% | Haringey | 84% |
| Tower Hamlets | 65% | Havering | 73% | Newham | 86% |

Exported Demand

Definition of exported demand – The residue of satisfied demand, after retained demand, is exported demand.  This is based on London residents who live within the travel time of a sports hall located outside London and use that sports hall.

- Only 3.6% of the London Region’s satisfied demand is exported outside London. This equates to 22,709 visits in the weekly peak period.

- At borough and regional level, the data from the National FPM Run does not identify how much of London’s demand goes to other adjoining local authority areas or individual sports halls but only provides the total figure for exported satisfied demand.  However, it can be concluded that very low levels of demand for sports halls by London residents is met outside the London area.

- Within London, the percentages of exported satisfied demand do vary widely across the Boroughs. The Boroughs with the lowest retained demand have the highest exported demand to other London Boroughs.

- The findings on retained and exported demand for sports halls are important, as increased demand in one London Borough may have a significant impact on demand for the use of sports halls in an adjoining borough. Some boroughs consider it important to provide for and meet their residents demand for sports halls own borough, while others do not, so long as their demand for sports halls can be met somewhere. Some authorities welcome exported demand, from other authorities, if it increases the usage and income of their sports halls.  Others will consider that this places an unacceptable pressure on already busy facilities.

Table 5.6: Exported demand as a percent of total satisfied demand

| Borough | % | Borough | % | Borough | % |
| --- | --- | --- | --- | --- | --- |
| Newham | 14% | Havering | 27% | Tower Hamlets | 35% |
| Haringey | 16% | Hillingdon | 28% | Lambeth | 35% |
| Westminster | 17% | Brent | 29% | Merton | 36% |
| Waltham Forest | 19% | Richmond upon Thames | 30% | Southwark | 38% |
| Bexley | 20% | Wandsworth | 30% | Harrow | 40% |
| Bromley | 21% | Hackney | 31% | Camden | 41% |
| Sutton | 22% | Croydon | 31% | Ealing | 44% |
| Islington | 24% | 31% | 31% | Lewisham | 53% |
| Kingston upon Thames | 25% | Greenwich | 32% | Kensington & Chelsea | 55% |
| Barnet | 26% | Hounslow | 35% | Hammersmith & Fulham | 77% |
| Redbridge | 27% | Barking & Dagenham | 35% | City of London | 100% |

## Section 6: Unmet Demand

Demand from residents not currently being met

Table 6.1 Unmet Demand for Sports Halls in 2025

| Unmet Demand for Sports Halls | London total |
| --- | --- |
| Total number of visits in the peak, not currently being met (visits per week peak period) | 70,463 |
| Unmet demand as a percentage of total demand | 10% |
| Equivalent in Courts - with comfort factor | 239 |
| Percentage of 10% most deprived demand unmet | 13% |
| Percentage of unmet demand due to |  |
| Lack of facility capacity | 48% |
| Facility too far away | 52% |

Definition of unmet demand – This has two parts, demand for sports halls that cannot be met because either:

- There is too much demand for any particular sports hall within its travel time area and there is lack of capacity.

- The demand is located too far from any sports halls that it can use (taking into account deprivation) or reach (taking into account transport access) and is then classified as demand.

Analysis of the spread of unmet demand shows the level of unmet demand that would be met by a potential new facility in any given location.  This ‘reachable unmet demand’ is calculated for each one-kilometre grid square, and figures are shown in the map.

- The total weekly demand for visits to sports halls in the peak period across London is 709,045.  638,582 of those visits demanded are satisfied, mostly within the London region, with a small percentage being exported to local authority areas adjoining London boroughs. Unmet demand accounts for 10% of London’s total demand and equates to 70,463 visits in the weekly peak period.  As shown in Table 6.1 above the level of unmet demand in top 10% most deprived areas in London (13%) is significantly higher than the London average (10%).

- The difference between the total supply of sports halls and the effective supply in 2025 is 757 badminton courts or 28.9% of the total supply in badminton courts. Or put another way, 189 sports halls if the supply was uniformly in 4 badminton court size sports halls. Therefore, the unavailable supply in 2025 of 757 badminton courts exceeds the total unmet demand of 239.3 courts in 2025.

- The total unmet demand for sports halls across London is 10% of total demand in 2025. This equates to 70,243 visits per week in peak hours or 239.3 badminton courts. For context London has a total supply of 1,859 badminton courts available for community use in the peak period in 2025.

Causes of Unmet Demand

- A key finding is that unmet demand totals 239 courts. Residents who are too far from a sports hall account for 52% of this unmet demand. The remaining 48% of unmet demand is due to the lack of facility capacity.

- Applying the National Model, most of the unmet demand comes from those without access to a car. This is possibly less relevant in London given high levels of public transport in many areas but highlights the importance of ensuring that facilities are accessible by public transport, walking and cycling.

- Unmet demand that is located too far from a sports hall will always exist because it is not possible to achieve complete spatial coverage whereby all areas of London area are within walking distance of a sports hall that is not commercial and not everyone will want or is able to drive.

- The key point here is to consider the scale of unmet demand.  Also, if it is clustered in one location, further provision should be considered to improve accessibility.

Geographical Distribution

- The locations with the highest amounts of unmet demand in 2025 are located in the London Boroughs of Hammersmith and Fulham, on the border between Westminster and Camden, Southwark and Tower Hamlets (see dark red squares on Map 6.1).  In general, the areas of greatest unmet need are in the inner London boroughs, with the outer London Boroughs having very low levels of unmet need.

- The distribution of unmet demand for all Boroughs is set out in Table 6.2 with the five highest and lowest levels of unmet demand and expressed in badminton courts highlighted in green and pink respectively. Table 6.2 shows that there is minimal unmet demand in some boroughs but significant unmet demand in others.  There are only 8 boroughs where overall demand is below 4 courts.

Definition of reachable unmet demand – Analysis of the spread of unmet demand shows the level of unmet demand that would be met by a potential new facility in any given location.  This ‘reachable unmet demand’ is calculated for each one-kilometre grid square, and figures are shown in the map.

Meeting Unmet Demand

- A key finding is that the location where the most unmet demand can be met in London is in Earls Court at 7.2 courts (dark orange square in Map 6.2).  There is sufficient unmet demand in this part of west London to justify a new sports hall at or near to this location. There is also sufficient demand (more than 4 court equivalent reachable demand in several kilometre squares) in other inner London locations including Southwark, northwest Lewisham, north Wandsworth and Lambeth, most of Hammersmith and Fulham and Kensington and Chelsea, east Ealing, and southeast Brent where further provision could be justified.

- This finding does not cover future changes in demand, which would need to be considered separately in a bespoke report.

- Some of the unmet demand could be met by opening up the venues on school, college and higher education sites where currently there is no or reduced access for community use. The scale of the findings on the potential supply does, however, indicate there is a need to also increase the provision of sports halls.

- As set out under the supply heading, the total supply of sports halls equates to 2,616 badminton courts in 2025. When the supply is assessed based on the supply available for community use in the weekly peak period, this reduces to 1,859 badminton courts. The reason for the difference between the total and effective supply is primarily because of the reduced hours for community use at sports halls on education sites.

Map 6.1: Unmet demand for sports halls as shown in number of courts

Map 6.2: Reachable Unmet Demand for Sports Halls as shown in number of courts

Table 6.2: Unmet Demand for Sports Halls equivalent in courts – with comfort factor in 2025

| Borough | Number of courts | Borough | Number of courts | Borough | Number of courts |
| --- | --- | --- | --- | --- | --- |
| City of London | 0.5 | Islington | 4.7 | Croydon | 8.8 |
| Sutton | 1.0 | Enfield | 4.9 | Greenwich | 9.1 |
| Kingston upon Thames | 1.3 | Hillingdon | 5.6 | Kensington & Chelsea | 10.6 |
| Bexley | 1.9 | Barking & Dagenham | 6.0 | Lambeth | 11.2 |
| Havering | 2.5 | Haringey | 6.5 | Wandsworth | 11.7 |
| Richmond upon Thames | 2.6 | Barnet | 6.6 | Brent | 12.8 |
| Redbridge | 3.6 | Westminster | 7.0 | Tower Hamlets | 13.8 |
| Harrow | 3.9 | Newham | 7.6 | Lewisham | 13.8 |
| Merton | 4.0 | Camden | 7.8 | Ealing | 14.2 |
| Bromley | 4.1 | Hackney | 7.9 | Southwark | 15.5 |
| Waltham Forest | 4.4 | Hounslow | 8.1 | Hammersmith & Fulham | 15.5 |

## Section 7: Used Capacity

How well used are the facilities?

Table 7.1: Used Capacity of Sports Halls in 2025

| Used Capacity of Sports Halls | London total |
| --- | --- |
| Total number of visits used of current capacity (visits per week peak period) | 631,116 |
| % of overall capacity of halls used | 92% |
| Visits Imported |  |
| Number of visits imported per week in peak period | 15,244 |
| Demand imported as a % of used capacity | 2% |
| Difference between visits imported and exported | -7,466 |

Definition of used capacity – This is a measure of usage at sports halls and estimates how well used or full facilities are.  The FPM is designed to include a ‘comfort factor’, beyond which the venues are too full.  The sports hall itself becomes too crowded to participate comfortably, and the changing and circulation areas also become too congested.  In the model Sport England assumes that usage of more than 80% of capacity is busy and that the sports hall is operating at an uncomfortable level above that percentage.

The summary of findings on used capacity is:

- In 2025 the London average for sports hall capacity used is 92% in the weekly peak period. This figure is significantly over the Sport England sports hall full comfort level of 80% of capacity used at peak times.  The reason the London average used capacity is so high is because demand is considerably greater than supply.  This means that residents may find it difficult to book a sports hall.   It is another finding that underlines the importance of increasing access to the sports halls on education sites where there is currently limited access for community use.  The difference between the total supply of sports halls and the effective supply in 2025 is 757 badminton courts or 28.9% of the total supply in badminton courts, or, equivalent to 189 sports halls if the supply was in 4 badminton court size sports halls.

- Increasing access to these education sports halls would reduce considerably the London wide average for used capacity of sports halls. It would create a more even and lower distribution of demand, whilst also reducing the number of new sports halls required. There is however also a need to increase the provision of sports halls as there is a lack of capacity overall and education halls are not available during core school hours and therefore cannot be relied on to address identified deficits.

- The used capacity of individual sports halls does vary from the Borough wide average which can be misleading when looking at what is happening at the individual sports hall sites in a Borough.  In a more detailed study at a sub area or for an individual Borough it would be important to look at the data for each sports hall. In a London wide study this is not possible, given there are 481 sports hall sites and 684 individual sports halls. Appendix 1 sets out the level of used and unused capacity for all the sports halls included in the study. The factors influencing used capacity are:

Type of site operator (public/educational/commercial)

- Public leisure centres have a ‘draw effect’ because they

- Are accessible for public and sports club use

- Have extensive opening hours and are proactively managed to encourage and support participation and physical activity

- Unlike commercial facilities, do not require payment of a monthly membership fee

- Provide all activities

- Access to sports halls for community use will be determined by the policy of each educational provider.

- Some schools and colleges actively promote community use

- At some venues there is little differentiation between educational and wider community use, with community access based on a membership system (classed as commercial)

- Other educational venues let out their sports halls to sports clubs or community groups on a termly basis, or for shorter periods

- A sports hall on an educational site that is only available for a few hours a week, and with an irregular pattern of use, is very different from a public leisure centre sports halls with a full programme of use.  Also, educational venues will not be available for recreational pay-and-play.

- Commercial sites provide recreational participation through membership.  These centres are not available for public pay-and-play or for club development.  Therefore, they offer a more limited programme of use than public leisure centres and are less utilised.

Age of the facility and its ‘attractiveness’

- To assess their comparative attractiveness to customers, all the sports hall sites in the model are weighted to reflect their age, operation type, condition and whether they have been modernised.

- The effect of refurbishment at a site decreases as the site gets older, and it becomes less attractive than a site built in the same year as the refurbishment.

Location of demand and competition from other sites

- Where sports halls are located close together, the demand that can reach these sites is shared between the venues, and this contributes to the level of used capacity at each.

- Conversely, where a sports hall is remotely located, it benefits from having no competition for the local demand.

Capacity

- When reviewing the estimated used capacity, it is important to consider the capacity of the site and not just the proportion in isolation.  Centres with the same or similar proportions of used capacity can accommodate very different levels of demand.

- The hours when a site is available for community use affects its capacity at peak times.

- Table 7.2 sets out the level of used capacity for all Boroughs with the highest and lowest levels of used capacity highlighted in green and pink respectively.  As the table illustrates there are 10 Boroughs where the estimated used capacity of the sports halls is 100%. In these authorities a reduction in used capacity cannot be created unless there is increased access to the existing sports halls, where there is limited access, or more sports halls are provided.  In summary, 28 out of 33 London boroughs have used capacity levels higher than the 80% comfort level, with a further three boroughs at 80%. This impacts on the quality of the user experience.

Table 7.2: Used Capacity of Sports Halls as a percentage of total capacity across all London boroughs in 2025

| Borough | Percentage of total capacity | Borough | Percentage of total capacity | Borough | Percentage of total capacity |
| --- | --- | --- | --- | --- | --- |
| City of London | 0.0 | Hillingdon | 93% | Barking & Dagenham | 99% |
| Havering | 70% | Greenwich | 93% | Camden | 100% |
| Bromley | 80% | Haringey | 94% | Islington | 100% |
| Kingston upon Thames | 80% | Enfield | 94% | Kensington & Chelsea | 100% |
| Sutton | 80% | Harrow | 95% | Lambeth | 100% |
| Redbridge | 85% | Waltham Forest | 95% | Westminster | 100% |
| Newham | 87% | Barnet | 95% | Hackney | 100% |
| Richmond upon Thames | 88% | Hounslow | 97% | Lewisham | 100% |
| Merton | 90% | Southwark | 97% | Wandsworth | 100% |
| Croydon | 90% | Tower Hamlets | 98% | Ealing | 100% |
| Bexley | 90% | Brent | 98% | Hammersmith & Fulham | 100% |

Imported Demand

- If residents of neighbouring local authority areas participate at a site within London, their usage becomes part of the used capacity of the London’s sports halls.

- Imported demand from outside of London area accounts for 2% of the used capacity of London sports halls.  The sports halls meet 15,244 visits in the weekly peak period from residents of neighbouring local authorities.

- London exports 7,465 more visits in the weekly peak period than it imports.  The total level of imports across all London boroughs is shown in the table 7.3.

- The data from the National FPM Run does not identify how much demand from each neighbouring local authority is imported and met at London sports halls and only provides the total figure for imported demand.  The amount of imported demand from each individual local authority could be identified in a bespoke FPM run.

- Imported demand is measured under used capacity because if the nearest sports hall for a resident in authority A is a sports hall in authority B, and they use the nearest venue to where they live, then this becomes part of the used capacity of sports halls in authority B

- The imported demand for individual Boroughs is set out in Table 7.3 with the five highest and lowest levels of imported demand highlighted in green and pink respectively. The boroughs with high imported demand, reflect that the sports hall locations and catchment area of sports halls in their borough overlap extensively with sports halls in neighbouring authorities. The data is indicating that there is a lot of movement across London Borough boundaries.

Table 7.3: Imported Demand as a percentage of used capacity of spots halls in 2025

| Borough | Percentage of total capacity | Borough | Percentage of total capacity | Borough | Percentage of total capacity |
| --- | --- | --- | --- | --- | --- |
| City of London | 0 | Hillingdon | 27% | Southwark | 34% |
| Tower Hamlets | 11% | Barking & Dagenham | 30% | Bexley | 35% |
| Havering | 13% | Redbridge | 31% | Hammersmith & Fulham | 35% |
| Enfield | 19% | Brent | 31% | Hackney | 35% |
| Ealing | 20% | Greenwich | 31% | Richmond upon Thames | 36% |
| Newham | 22% | Waltham Forest | 32% | Sutton | 37% |
| Barnet | 23% | Kingston upon Thames | 32% | Islington | 40% |
| Camden | 24% | Bromley | 33% | Merton | 40% |
| Harrow | 24% | Lewisham | 33% | Kensington & Chelsea | 42% |
| Croydon | 25% | Haringey | 33% | Westminster | 42% |
| Wandsworth | 26% | Lambeth | 34% | Hounslow | 44% |

## Section 8: Local Share of Facilities

Equity share of facilities

Table 8.1: Local Share of Sports Halls for London in 2025

| Local Share of Sports Halls for London | London total |
| --- | --- |
| Local share of sports halls relative to demand in local area: <1 = poorer, >1 = better | 0.51 |
| Courts per 10,000 population | 2.9 |

Definition of local share – This helps to show which areas have a better or worse share of facility provision.  It considers the size, availability and quality of facilities, as well as travel modes.  Local share is useful for looking at ‘equity’ of provision.  Local share is the available capacity that people want to visit in an area, divided by the demand for that capacity in the area (considering deprivation).  Local share decreases as facilities age.

Share of Supply

  - Local share shows how the access and share of sports halls differs across the study area, as follows:

- A value of 1 means that there is sufficient suitable supply reachable by the demand. The score of 1 is set as the England wide average of local share

- A value of less than 1 indicates a shortage of suitable supply that can be reached by the demand

- A value greater than 1 indicates a surplus of suitable supply that can be reached by the demand

  - Overall, local share identifies the areas of the study area where the share of sports halls is better and worse.  The intervention is to try and increase access for residents in the areas with the poorest access to sports halls.

  - A key finding is that London has a local share of 0.5 and so supply is significantly less than demand across London, when compared to England as a whole and there is no London Borough where local share meets or exceeds the average share across England (see table 8.2).

  - Local share is worse in the inner London Boroughs and comparatively better in the outer ones (see Map 8.1). The areas of worse share are south Hammersmith and Fulham (6 km squares), Kensington and Chelsea (4 km squares), south Harrow (6 km squares), St James’s and Fitzrovia in Westminster (2 km squares), northeast Southwark (9 km squares) and northwest Lewisham (15 km squares), the Greenwich Peninsula (1 km square), and northeast Newham (2 km squares)

Table 8.2: Local Share of Sports Halls relative to demand across all boroughs in 2025 (<1 = poorer, >1 = better)

| Borough | Local Share | Borough | Local Share | Borough | Local Share |  |
| --- | --- | --- | --- | --- | --- | --- |
| Hammersmith & Fulham | 0.35 | Greenwich | 0.47 | Newham |  | 0.56 |
| Lewisham | 0.37 | Hackney | 0.48 | Hillingdon |  | 0.56 |
| Southwark | 0.39 | Enfield | 0.49 | Bromley |  | 0.56 |
| Kensington & Chelsea | 0.43 | Tower Hamlets | 0.49 | Westminster |  | 0.57 |
| Wandsworth | 0.43 | Islington | 0.49 | Kingston upon Thames |  | 0.58 |
| Harrow | 0.45 | Barking & Dagenham | 0.49 | Bexley |  | 0.59 |
| Haringey | 0.45 | Merton | 0.50 | Sutton |  | 0.59 |
| Brent | 0.46 | Barnet | 0.51 | Richmond upon Thames |  | 0.60 |
| Ealing | 0.46 | City of London | 0.51 | Havering |  | 0.61 |
| Lambeth | 0.47 | Hounslow | 0.55 | Redbridge |  | 0.63 |
| Camden | 0.47 | Croydon | 0.55 | Waltham Forest |  | 0.72 |

Comparative Measure of Provision

  - A comparative measure for assessing supply of sports halls across each of the boroughs authorities on a consistent basis is badminton courts per 10,000 population. Across London there are 2.9 badminton courts per 10,000 population in 2025. The England wide findings are 4.3 badminton courts per 10,000 population in 2025. Table 8.3 shows the ratio of courts per 10,000 population for each Borough. Only Sutton meets and marginally exceeds the England average.

  - The findings on badminton courts per 10,000 population are reported because some local authorities like to compare their quantitative provision with others; however, it does not set a standard of provision and should not be used as such.

Table 8.3: Number of badminton courts available per 10,000 population across all Boroughs

| Borough | Number of courts | Borough | Number of courts | Borough | Number of courts |
| --- | --- | --- | --- | --- | --- |
| City of London | 0.0 | Barking & Dagenham | 2.6 | Islington | 3.2 |
| Hammersmith & Fulham | 0.9 | Southwark | 2.7 | Havering | 3.3 |
| Ealing | 1.7 | Hillingdon | 2.7 | Waltham Forest | 3.4 |
| Kensington & Chelsea | 1.7 | Lambeth | 2.8 | Haringey | 3.5 |
| Tower Hamlets | 1.9 | Hackney | 2.8 | Bromley | 3.7 |
| Lewisham | 1.9 | Croydon | 2.9 | Redbridge | 3.7 |
| Camden | 2.1 | Hounslow | 2.9 | Newham | 3.9 |
| Harrow | 2.4 | Greenwich | 2.9 | Bexley | 3.9 |
| Wandsworth | 2.4 | Barnet | 2.9 | Richmond upon Thames | 3.9 |
| Brent | 2.5 | Merton | 3.0 | Kingston upon Thames | 4.1 |
| Enfield | 2.5 | Westminster | 3.2 | Sutton | 4.4 |

Map 8.1: Local share of sports halls London 2025

This ends the reporting of the full findings on the London assessment of sports halls provision from the 2025 FPM run. The key findings are set out in the Executive Summary.

## Appendix 1: Sports Halls included in the assessment 2025

The following Table contains all the individual sports halls included in the assessment.   This information is taken from Active Places in March 2025.  An explanation of the model, inclusion criteria and model parameters is given in Appendix 3.

The audit excludes facilities that are deemed to be either for private use, too small, closed or there is a lack of information, particularly relating to hours of use.  For more information about excluded facilities please see Appendix 3 (Facility Inclusion Criteria).

In January 2026 the Greater London Authority contacted all local authorities with a copy of the Active Places data to identify any changes that had occurred since March 2025.   Where these occur, they are recorded in the bottom table for information, and it is important to note that this information has not informed the production of this report.  Some London Boroughs have also undertaken these reports at a local level. The reports for those boroughs may differ in terms of the facilities included.   Where decisions are being made by Boroughs about facility development, the local report should be referred to over this report.

The outputs from this report have been prepared to give a strategic overview of sports hall supply in London.  Longer-term local bespoke assessments can be undertaken using Sport England’s FPM.  These assessments could include population projections with options for changing the swimming pool/sports hall supply and assessing the collective impact this has on the future demand for swimming pools/sports halls and the distribution of that demand.

| Name of Site | Type | Dimensions | Area | Site Year Built | Site Year Refurb | % of Capacity Not Used | % of Capacity Used | % of Capacity Not Used | Car % Demand | Public Transport % Demand | Walk % Demand |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Camden |  |  |  |  |  |  | 100% | 0% | 38% | 16% | 46% |
| ACLAND BURGHLEY SCHOOL | 4-court | 34.5 x 20 | 690 | 1965 | 2006 | Educational (in-house) | 100% |  | 34% | 13% | 53% |
| ACLAND BURGHLEY SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| KAJIMA COMMUNITY AT HAVERSTOCK SCHOOL | 4-court | 33 x 18 | 594 | 2005 |  | Educational (in-house) | 100% |  | 37% | 15% | 48% |
| REGENT HIGH SCHOOL | 3-court | 27 x 18 | 486 | 1959 | 2014 | Educational (in-house) | 100% |  | 26% | 15% | 59% |
| REGENT HIGH SCHOOL | 3-court | 27 x 18 | 486 |  |  |  |  |  |  |  |  |
| REGENT HIGH SCHOOL | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| ROYAL FREE HOSPITAL RECREATION CLUB | 4-court | 33 x 18 | 594 | 1978 | 2006 | Other (pay & play) | 100% |  | 45% | 16% | 39% |
| SOMERS TOWN COMMUNITY SPORTS CENTRE | 6-court | 32 x 28 | 896 | 1997 | 2012 | Educational (in-house) | 100% |  | 27% | 16% | 57% |
| SOUTH HAMPSTEAD HIGH SCHOOL | 4-court | 33 x 20 | 660 | 2014 |  | Educational (in-house) | 100% |  | 48% | 17% | 35% |
| SWISS COTTAGE LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2006 | 2015 | Public | 100% |  | 47% | 18% | 35% |
| TALACRE COMMUNITY SPORTS CENTRE | 4-court | 34.5 x 20 | 690 | 2002 | 2004 | Public | 100% |  | 34% | 14% | 52% |
| UCS ACTIVE | 4-court | 34.5 x 20 | 690 | 2007 |  | Public | 100% |  | 49% | 16% | 35% |
| Islington |  |  |  |  |  |  | 100% | 0% | 36% | 20% | 44% |
| BEACON HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 2007 | 2008 | Educational (in-house) | 100% |  | 31% | 15% | 55% |
| CITY OF LONDON ACADEMY | 4-court | 34.5 x 20 | 690 | 2009 |  | Educational (in-house) | 100% |  | 30% | 15% | 55% |
| CITY OF LONDON ACADEMY HIGHBURY GROVE | 4-court | 33 x 18 | 594 | 2010 |  | Educational (in-house) | 100% |  | 47% | 24% | 29% |
| CITY OF LONDON ACADEMY HIGHBURY GROVE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CITY OF LONDON ACADEMY HIGHGATE HILL | 3-court | 27 x 18 | 486 | 1960 |  | Educational (in-house) | 100% |  | 27% | 11% | 62% |
| CITY OF LONDON ACADEMY HIGHGATE HILL | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| CITYSPORT | 6-court | 34.5 x 27 | 932 | 2015 |  | Educational (in-house) | 100% |  | 37% | 23% | 40% |
| ELIZABETH GARRETT ANDERSON SCHOOL | 4-court | 34.5 x 20 | 690 | 1960 | 2001 | Educational (in-house) | 100% |  | 26% | 16% | 58% |
| FINSBURY LEISURE CENTRE | 4-court | 32.1 x 21.25 | 682 | 1975 | 2004 | Public | 100% |  | 35% | 21% | 44% |
| FINSBURY LEISURE CENTRE | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| FINSBURY LEISURE CENTRE | Activity | 24 x 16 | 384 |  |  |  |  |  |  |  |  |
| FINSBURY LEISURE CENTRE | Activity | 24 x 16 | 384 |  |  |  |  |  |  |  |  |
| ISLINGTON ARTS & MEDIA SCHOOL | 4-court | 34.5 x 20 | 690 | 2003 |  | Educational (in-house) | 100% |  | 37% | 17% | 46% |
| LONDON METROPOLITAN UNIVERSITY (HOLLOWAY CAMPUS) | 6-court | 33 x 27 | 891 | 2006 |  | Educational (in-house) | 100% |  | 41% | 21% | 38% |
| LONDON METROPOLITAN UNIVERSITY (HOLLOWAY CAMPUS) | 3-court | 27 x 18 | 486 |  |  |  |  |  |  |  |  |
| SOBELL LEISURE CENTRE (HOLLOWAY) | 8-court | 37 x 34 | 1258 | 1972 | 2017 | Public | 100% |  | 33% | 17% | 50% |
| ST MARY MAGDALENE ACADEMY | 4-court | 34.5 x 20 | 690 | 2005 |  | Educational (in-house) | 100% |  | 38% | 20% | 42% |
| ST MARY MAGDALENE ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| Kensington & Chelsea |  |  |  |  |  |  | 100% | 0% | 49% | 16% | 35% |
| CHELSEA ACADEMY | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (in-house) | 100% |  | 34% | 9% | 57% |
| HARROW CLUB W10 | 3-court | 27 x 18 | 486 | 1965 | 2015 | Other (sport club) | 100% |  | 49% | 16% | 35% |
| KENSINGTON ALDRIDGE ACADEMY | 4-court | 34.5 x 20 | 690 | 2014 |  | Educational (in-house) | 100% |  | 52% | 18% | 30% |
| KENSINGTON LEISURE CENTRE | 8-court | 37 x 33 | 1221 | 2015 |  | Public | 100% |  | 50% | 17% | 33% |
| ST CHARLES CATHOLIC SIXTH FORM COLLEGE | 4-court | 33 x 18 | 594 | 2009 |  | Educational (in-house) | 100% |  | 50% | 17% | 33% |
| ST CHARLES CATHOLIC SIXTH FORM COLLEGE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| Lambeth |  |  |  |  |  |  | 100% | 0% | 46% | 16% | 39% |
| ARK EVELYN GRACE ACADEMY | 4-court | 33 x 18 | 594 | 2010 |  | Educational (in-house) | 100% |  | 49% | 19% | 31% |
| BISHOP THOMAS GRANT SCHOOL | 4-court | 33 x 18 | 594 | 1971 | 2015 | Educational (in-house) | 100% |  | 60% | 16% | 24% |
| BISHOP THOMAS GRANT SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BISHOP THOMAS GRANT SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BLACK PRINCE COMMUNITY HUB | 4-court | 34.5 x 20 | 690 | 1964 | 2022 | Public | 100% |  | 38% | 18% | 44% |
| BLACK PRINCE COMMUNITY HUB | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| BRIXTON RECREATION CENTRE | 8-court | 37 x 33 | 1221 | 1985 | 2003 | Public | 100% |  | 42% | 18% | 41% |
| BRIXTON RECREATION CENTRE | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| CITY HEIGHTS E-ACT ACADEMY | 4-court | 34.5 x 20 | 690 | 2014 |  | Educational (in-house) | 100% |  | 35% | 12% | 53% |
| CLAPHAM LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2012 |  | Public | 100% |  | 41% | 15% | 44% |
| DUNRAVEN SCHOOL (SOUTH SIDE) | 4-court | 34.5 x 20 | 690 | 2009 |  | Educational (in-house) | 100% |  | 45% | 13% | 42% |
| FLAXMAN SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1976 | 2018 | Public | 100% |  | 42% | 17% | 41% |
| LA RETRAITE ROMAN CATHOLIC GIRLS SCHOOL | 4-court | 33 x 18 | 594 | 2010 |  | Educational (3rd party) | 100% |  | 35% | 10% | 55% |
| LILIAN BAYLIS TECHNOLOGY SCHOOL | 4-court | 33 x 18 | 594 | 2005 | 2007 | Educational (in-house) | 100% |  | 39% | 18% | 43% |
| LONDON NAUTICAL SCHOOL | 4-court | 34.5 x 20 | 690 | 2005 |  | Educational (in-house) | 100% |  | 34% | 17% | 50% |
| NORWOOD SCHOOL | 4-court | 33 x 18 | 594 | 1970 | 2012 | Educational (in-house) | 100% |  | 60% | 15% | 25% |
| NORWOOD SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| PLATANOS COLLEGE | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (in-house) | 100% |  | 34% | 15% | 51% |
| SOUTH BANK COLLEGES (CLAPHAM CENTRE) | 4-court | 33 x 18 | 594 | 2015 |  | Educational (in-house) | 100% |  | 47% | 15% | 39% |
| STREATHAM & CLAPHAM HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 1997 |  | Educational (3rd party) | 100% |  | 50% | 14% | 36% |
| STREATHAM ICE AND LEISURE CENTRE | 4-court | 34.5 x 20 | 690 | 2013 |  | Public | 100% |  | 51% | 13% | 36% |
| THE ELMGREEN SCHOOL | 4-court | 33 x 18 | 594 | 2009 |  | Educational (in-house) | 100% |  | 49% | 15% | 36% |
| THE ELMS ACADEMY | 4-court | 33 x 18 | 594 | 2004 |  | Educational (3rd party) | 100% |  | 46% | 14% | 40% |
| Southwark |  |  |  |  |  |  | 97% | 3% | 48% | 16% | 36% |
| ARK ALL SAINTS ACADEMY | 4-court | 33 x 18 | 594 | 2014 |  | Educational (in-house) | 100% |  | 39% | 16% | 45% |
| ARK ALL SAINTS ACADEMY | Activity | 19.3 x 11.5 | 222 |  |  |  |  |  |  |  |  |
| ARK GLOBE ACADEMY | 4-court | 33 x 18 | 594 | 2010 | 2012 | Educational (in-house) | 100% |  | 26% | 12% | 62% |
| ARK WALWORTH ACADEMY | 4-court | 33 x 18 | 594 | 2010 |  | Educational (in-house) | 100% |  | 26% | 10% | 63% |
| BACONS COLLEGE SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1992 | 2014 | Educational (in-house) | 100% |  | 46% | 14% | 40% |
| CHARTER SCHOOL EAST DULWICH | 4-court | 34.5 x 20 | 690 | 2018 |  | Educational (in-house) | 100% |  | 32% | 9% | 59% |
| CHARTER SCHOOL SPORTS CENTRE | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 100% |  | 54% | 17% | 29% |
| DAMILOLA TAYLOR YOUTH CENTRE | 3-court | 27 x 18 | 486 | 2001 | 2010 | Public | 100% |  | 29% | 10% | 60% |
| DULWICH COLLEGE SPORTS CLUB | 7-court | 36.6 x 30.5 | 1116 | 1960 |  | Educational (in-house) | 81% | 19% | 68% | 19% | 13% |
| DULWICH COLLEGE SPORTS CLUB | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| DULWICH COLLEGE SPORTS CLUB | Activity | 17.5 x 9 | 158 |  |  |  |  |  |  |  |  |
| DULWICH PREP LONDON | 4-court | 33 x 18 | 594 | 2006 |  | Educational (in-house) | 100% | 0% | 59% | 16% | 25% |
| HABERDASHERS ASKES HATCHAM COLLEGE (NUNHEAD SPORTS FIELD) | 4-court | 33 x 18 | 594 | 2003 |  | Educational (in-house) | 100% |  | 43% | 13% | 43% |
| HARRIS ACADEMY PECKHAM | 4-court | 34.5 x 20 | 690 | 2006 |  | Educational (in house) | 100% |  | 32% | 11% | 58% |
| HARRIS FITNESS CENTRE (EAST DULWICH) | 4-court | 34.5 x 20 | 690 | 2004 |  | Educational (in-house) | 100% |  | 49% | 14% | 37% |
| JAGS SPORTS CLUB | 4-court | 34.5 x 20 | 690 | 1975 | 1999 | Educational (3rd party) | 100% |  | 58% | 18% | 24% |
| KINGSDALE FOUNDATION SCHOOL | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 100% |  | 63% | 17% | 20% |
| LONDON SOUTH BANK UNIVERSITY SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1975 | 2022 | Educational (in-house) | 100% |  | 33% | 16% | 51% |
| SACRED HEART ROMAN CATHOLIC SECONDARY SCHOOL | 6-court | 34.5 x 27 | 932 | 2014 |  | Educational (in-house) | 100% |  | 44% | 18% | 37% |
| THE CASTLE CENTRE | 4-court | 34.5 x 20 | 690 | 2016 |  | Public | 100% |  | 26% | 12% | 61% |
| THE CITY OF LONDON ACADEMY (SOUTHWARK) | 4-court | 33 x 18 | 594 | 2005 |  | Educational (in-house) | 100% |  | 33% | 12% | 55% |
| THE SALMON YOUTH CENTRE | 4-court | 33 x 18 | 594 | 2007 |  | Other (pay & play) | 100% |  | 24% | 10% | 66% |
| Westminster |  |  |  |  |  |  | 100% | 0% | 43% | 16% | 42% |
| ACADEMY SPORT | 4-court | 33 x 18 | 594 | 2007 |  | Educational (3rd party) | 100% |  | 48% | 20% | 33% |
| AMERICAN SCHOOL IN LONDON | 4-court | 33 x 18 | 594 | 2016 |  | Educational (in-house) | 100% |  | 44% | 16% | 39% |
| CHURCH STREET COMMUNITY LEISURE CENTRE | 3-court | 27 x 18 | 486 | 2023 |  | Public | 100% |  | 39% | 16% | 46% |
| CITY OF WESTMINSTER COLLEGE | 4-court | 33.2 x 18.1 | 601 | 2011 |  | Educational (in-house) | 100% |  | 36% | 15% | 49% |
| HARRIS ACADEMY ST JOHN'S WOOD | 3-court | 28 x 18 | 504 | 1995 |  | Educational (in-house) | 100% |  | 46% | 17% | 38% |
| JUBILEE COMMUNITY LEISURE CENTRE | 3-court | 27 x 18 | 486 | 2023 |  | Public | 100% |  | 38% | 13% | 49% |
| KING SOLOMON ACADEMY | 3-court | 27 x 18 | 486 | 2009 |  | Educational (in-house) | 100% |  | 41% | 17% | 42% |
| LITTLE VENICE SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2009 |  | Public | 100% |  | 44% | 17% | 40% |
| MOBERLY SPORTS CENTRE | 8-court | 37 x 34 | 1258 | 2018 |  | Public | 100% |  | 49% | 17% | 34% |
| PADDINGTON ACADEMY | 4-court | 33 x 18 | 594 | 2007 |  | Educational (3rd party) | 100% |  | 43% | 16% | 41% |
| PIMLICO ACADEMY | 4-court | 34.5 x 20 | 690 | 2009 |  | Educational (in-house) | 100% |  | 43% | 16% | 42% |
| QUEEN MOTHER SPORTS CENTRE | 6-court | 33 x 27 | 891 | 1981 | 2014 | Public | 100% |  | 45% | 16% | 39% |
| SEYMOUR LEISURE CENTRE | 5-court | 43 x 27 | 1161 | 1920 |  | Public | 100% |  | 42% | 18% | 40% |
| ST AUGUSTINE'S SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2010 |  | Educational (3rd party) | 100% |  | 40% | 15% | 45% |
| ST GEORGE RC SCHOOL | 3-court | 27 x 18 | 486 | 2010 |  | Educational (in-house) | 100% |  | 32% | 12% | 56% |
| THE GREY COAT HOSPITAL SCHOOL (ST MICHAELS) | 3-court | 27 x 18 | 486 | 1955 |  | Educational (in-house) | 100% |  | 39% | 16% | 45% |
| WESTMINSTER CITY SCHOOL | 3-court | 27 x 18 | 486 | 2009 |  | Educational (in-house) | 100% |  | 36% | 13% | 51% |
| Barking & Dagenham |  |  |  |  |  |  | 99% | 1% | 64% | 11% | 25% |
| BARKING ABBEY SCHOOL (LONGBRIDGE CAMPUS) | 4-court | 33 x 18 | 594 | 2019 |  | Educational (in-house) | 100% |  | 64% | 11% | 25% |
| BARKING ABBEY SCHOOL (LONGBRIDGE CAMPUS) | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| BARKING AND DAGENHAM COLLEGE | 4-court | 33 x 18 | 594 | 1960 | 2022 | Educational (in-house) | 100% |  | 71% | 12% | 17% |
| BARKING SPORTHOUSE AND GYM | 5-court | 40.6 x 21.35 | 867 | 2012 |  | Other (pay & play) | 100% |  | 59% | 10% | 31% |
| BECONTREE HEATH LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2011 |  | Public | 100% |  | 64% | 11% | 25% |
| CASTLE GREEN LEISURE CENTRE | 4-court | 34.5 x 20 | 690 | 2005 |  | Educational (3rd party) | 100% |  | 65% | 11% | 24% |
| DAGENHAM PARK LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2007 |  | Educational (in-house) | 100% |  | 56% | 10% | 34% |
| EASTBROOK SCHOOL | 5-court | 45 x 18 | 810 | 1995 | 2003 | Educational (3rd party) | 100% |  | 65% | 11% | 23% |
| EASTBROOK SCHOOL | 3-court | 27 x 18 | 486 |  |  |  |  |  |  |  |  |
| EASTBURY COMMUNITY SCHOOL | 4-court | 34.5 x 20 | 690 | 1996 |  | Educational (3rd party) | 100% |  | 57% | 10% | 33% |
| EASTBURY COMMUNITY SCHOOL | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| ROBERT CLACK SCHOOL (UPPER SITE) | 4-court | 33 x 18 | 594 | 2004 |  | Educational (in-house) | 100% |  | 69% | 12% | 19% |
| SYDNEY RUSSELL LEISURE CENTRE | 6-court | 33 x 27 | 891 | 1993 |  | Educational (in-house) | 100% |  | 66% | 12% | 22% |
| SYDNEY RUSSELL LEISURE CENTRE | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| SYDNEY RUSSELL LEISURE CENTRE | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| YMCA (THAMES GATEWAY) | 4-court | 34.5 x 20 | 690 | 1969 | 2000 | Other (pay & play) | 94% | 6% | 66% | 10% | 24% |
| Bexley |  |  |  | 1998 |  |  | 90% | 10% | 73% | 11% | 16% |
| BETHS GRAMMAR SCHOOL | 4-court | 33 x 18 | 594 | 2000 |  | Educational (in-house) | 74% | 26% | 76% | 9% | 15% |
| BEXLEY GRAMMAR SCHOOL | 4-court | 33 x 18 | 594 | 1998 |  | Educational (in-house) | 91% | 9% | 75% | 11% | 14% |
| BEXLEY GRAMMAR SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BEXLEYHEATH ACADEMY | 4-court | 33 x 18 | 594 | 1955 | 2006 | Educational (in-house) | 50% | 50% | 72% | 9% | 18% |
| BEXLEYHEATH ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BEXLEYHEATH ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BLACKFEN SCHOOL FOR GIRLS | 4-court | 33 x 18 | 594 | 1991 |  | Educational (in-house) | 100% |  | 76% | 10% | 14% |
| CLEEVE PARK SCHOOL | 4-court | 34.5 x 20 | 690 | 1999 |  | Educational (in-house) | 87% | 13% | 77% | 11% | 12% |
| CLEEVE PARK SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CROOK LOG LEISURE CENTRE | 6-court | 51 x 18 | 918 | 2005 |  | Public | 100% |  | 79% | 11% | 11% |
| DANSON YOUTH TRUST | 3-court | 33 x 18 | 594 | 1974 |  | Public | 83% | 17% | 77% | 11% | 12% |
| DANSON YOUTH TRUST | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| DANSON YOUTH TRUST | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| ERITH LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2005 |  | Public | 100% |  | 66% | 11% | 23% |
| HARRIS ACADEMY FALCONWOOD | 4-court | 32.8 x 18 | 590 | 2011 |  | Educational (in-house) | 86% | 14% | 72% | 11% | 17% |
| HARRIS GARRARD ACADEMY | 3-court | 27 x 18 | 486 | 2003 |  | Educational (in-house) | 100% |  | 59% | 14% | 26% |
| HARRIS GARRARD ACADEMY | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| HURSTMERE SCHOOL | 4-court | 33 x 18 | 594 | 2003 |  | Educational (in-house) | 87% | 13% | 78% | 10% | 12% |
| HURSTMERE SCHOOL | Activity | 0 x 0 | 0 |  |  |  |  |  |  |  |  |
| HURSTMERE SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| LEIGH ACADEMY BEXLEY | 4-court | 34.5 x 20 | 690 | 1997 |  | Educational (in-house) | 100% |  | 66% | 10% | 24% |
| SIDCUP LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2008 |  | Public | 100% |  | 74% | 10% | 16% |
| ST COLUMBA'S CATHOLIC BOYS SCHOOL | 4-court | 33 x 18 | 594 | 2005 |  | Educational (in-house) | 100% |  | 71% | 9% | 20% |
| TOWNLEY GRAMMAR SCHOOL | 4-court | 34.5 x 20 | 690 | 2006 |  | Educational (in-house) | 100% |  | 75% | 9% | 15% |
| TRINITY SCHOOL (BELVEDERE) | 4-court | 34.5 x 20 | 690 | 2003 |  | Educational (3rd party) | 100% |  | 68% | 12% | 20% |
| TRINITY SCHOOL (BELVEDERE) | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| WELLING SCHOOL | 4-court | 34.5 x 20 | 690 | 2005 |  | Educational (in-house) | 100% |  | 73% | 13% | 13% |
| WELLING SCHOOL | 4-court | 34.5 x 20 | 690 |  |  |  |  |  |  |  |  |
| WELLING SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| Greenwich |  |  |  |  |  |  | 93% | 7% | 62% | 16% | 23% |
| ARK GREENWICH FREE SCHOOL | 3-court | 27 x 18 | 486 | 2012 |  | Other (sport club) | 100% |  | 64% | 16% | 20% |
| BLACKHEATH HIGH SCHOOL | 3-court | 27 x 18 | 486 | 2012 |  | Educational (in-house) | 100% |  | 45% | 13% | 42% |
| COLFE'S LEISURE CENTRE | 6-court | 33 x 27 | 891 | 1992 |  | Educational (3rd party) | 100% |  | 66% | 16% | 18% |
| ELTHAM HILL SCHOOL | 4-court | 34.5 x 20 | 690 | 1995 |  | Educational (in-house) | 100% |  | 68% | 15% | 17% |
| HARRIS ACADEMY GREENWICH | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 100% |  | 71% | 16% | 13% |
| HARRIS ACADEMY GREENWICH | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| PLUMSTEAD MANOR SCHOOL | 4-court | 34.5 x 20 | 690 | 2012 |  | Educational (in-house) | 100% |  | 50% | 15% | 35% |
| PLUMSTEAD MANOR SCHOOL | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| PLUMSTEAD MANOR SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| SHOOTERS HILL SIXTH FORM COLLEGE | 4-court | 34.5 x 20 | 690 | 1965 | 2008 | Educational (in-house) | 74% | 26% | 57% | 16% | 28% |
| SHOOTERS HILL SIXTH FORM COLLEGE | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| ST MARY MAGDALENE C OF E SCHOOL (PENINSULA SITE) | 3-court | 28 x 18 | 504 | 2018 |  | Educational (in-house) | 100% |  | 23% | 7% | 69% |
| ST PAULS ACADEMY | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (in-house) | 100% |  | 40% | 10% | 50% |
| STATIONERS CROWN WOODS ACADEMY | 6-court | 34.5 x 27 | 932 | 2011 |  | Educational (3rd party) | 100% |  | 79% | 13% | 7% |
| THE HALLEY ACADEMY | 4-court | 33 x 18 | 594 | 2017 |  | Educational (in-house) | 100% |  | 64% | 16% | 20% |
| THE HALLEY ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| THE HALLEY ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| THOMAS TALLIS SCHOOL | 6-court | 34.5 x 27 | 932 | 2011 |  | Educational (3rd party) | 100% |  | 67% | 17% | 16% |
| THOMAS TALLIS SCHOOL | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| THOMAS TALLIS SCHOOL | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| UNIVERSITY OF GREENWICH (AVERY HILL) | 4-court | 37 x 18 | 666 | 2006 |  | Educational (in-house) | 100% |  | 72% | 11% | 17% |
| WATERFRONT LEISURE CENTRE | 4-court | 40 x 18 | 720 | 1986 |  | Public | 100% |  | 42% | 15% | 43% |
| WOOLWICH POLYTECHNIC SCHOOL FOR BOYS | 3-court | 27 x 18 | 486 | 1970 |  | Educational (in-house) | 55% | 45% | 61% | 17% | 22% |
| WOOLWICH POLYTECHNIC SCHOOL FOR BOYS | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| WOOLWICH POLYTECHNIC SCHOOL FOR GIRLS | 4-court | 33 x 18 | 594 | 2019 |  | Educational (in-house) | 100% |  | 66% | 19% | 15% |
| Hackney |  |  |  | 1998 |  |  | 100% | 0% | 37% | 16% | 47% |
| BRIDGE ACADEMY | 4-court | 33 x 18 | 594 | 2008 |  | Educational (in-house) | 100% |  | 34% | 16% | 50% |
| BRITANNIA LEISURE CENTRE | 6-court | 33 x 27 | 891 | 2021 |  | Public | 100% |  | 38% | 19% | 43% |
| BSIX SIXTH FORM COLLEGE | 4-court | 33 x 18 | 594 | 2004 |  | Educational (in-house) | 100% |  | 34% | 16% | 50% |
| CLAPTON GIRLS ACADEMY | 3-court | 27 x 18 | 486 | 1975 |  | Educational (in-house) | 100% |  | 38% | 16% | 45% |
| CLISSOLD LEISURE CENTRE | 6-court | 34 x 27 | 918 | 2002 | 2007 | Public | 100% |  | 38% | 18% | 44% |
| HAGGERSTON SCHOOL | 3-court | 27 x 18 | 486 | 1975 | 2012 | Educational (in-house) | 100% |  | 35% | 16% | 49% |
| HAGGERSTON SCHOOL | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| KINGS HALL LEISURE CENTRE | 3-court | 27 x 18 | 486 | 1984 | 2022 | Public | 100% |  | 35% | 15% | 50% |
| MOSSBOURNE VICTORIA PARK ACADEMY | 3-court | 27 x 18 | 486 | 2014 |  | Educational (in-house) | 100% |  | 37% | 15% | 48% |
| NEW CITY COLLEGE HACKNEY | 6-court | 33 x 27 | 891 | 2001 | 2012 | Educational (in-house) | 100% |  | 36% | 18% | 46% |
| PETCHEY ACADEMY | 4-court | 33 x 18 | 594 | 2007 |  | Educational (in-house) | 100% |  | 29% | 14% | 57% |
| QUEENSBRIDGE SPORTS & COMMUNITY CENTRE | 4-court | 33 x 18 | 594 | 1998 | 2010 | Public | 100% |  | 30% | 14% | 56% |
| STOKE NEWINGTON SCHOOL & SIXTH FORM | 4-court | 33 x 18 | 594 | 1950 | 2010 | Educational (in-house) | 100% |  | 38% | 18% | 44% |
| STOKE NEWINGTON SCHOOL & SIXTH FORM | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| THE CITY ACADEMY | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 100% |  | 27% | 11% | 61% |
| THE CITY ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| THE COPPER BOX ARENA | 10-court | 60 x 40 | 2400 | 2013 |  | Other (pay & play) | 100% |  | 46% | 18% | 36% |
| THE URSWICK SCHOOL | 4-court | 34.5 x 20 | 690 | 2011 |  | Educational (in-house) | 100% |  | 30% | 13% | 57% |
| Havering |  |  |  | 1987 |  |  | 70% | 30% | 73% | 9% | 18% |
| ABBS CROSS HEALTH AND FITNESS | 4-court | 33 x 18 | 594 | 2004 |  | Educational (3rd party) | 100% |  | 73% | 9% | 18% |
| BOWER PARK ACADEMY | 4-court | 33 x 18 | 594 | 1942 | 2007 | Educational (in-house) | 67% | 33% | 68% | 8% | 24% |
| CENTRAL PARK LEISURE CENTRE | 4-court | 34.5 x 20 | 690 | 2004 |  | Public | 100% |  | 67% | 9% | 24% |
| COOPERS COMPANY & COBORN SCHOOL | 4-court | 33 x 18 | 594 | 1971 |  | Educational (in-house) | 32% | 68% | 76% | 7% | 18% |
| COOPERS COMPANY & COBORN SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| DRAPERS ACADEMY | 4-court | 33 x 18 | 594 | 2012 |  | Educational (in-house) | 74% | 26% | 74% | 10% | 16% |
| EMERSON PARK ACADEMY | 4-court | 34.5 x 20 | 690 | 1968 | 2007 | Educational (in-house) | 45% | 55% | 83% | 8% | 9% |
| EMERSON PARK ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| GAYNES SCHOOL LANGUAGE COLLEGE | 4-court | 34.5 x 20 | 690 | 1965 | 2003 | Educational (in-house) | 35% | 65% | 78% | 7% | 15% |
| GAYNES SCHOOL LANGUAGE COLLEGE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HALL MEAD SCHOOL | 4-court | 34.5 x 20 | 690 | 1965 |  | Educational (in-house) | 32% | 68% | 73% | 7% | 20% |
| HALL MEAD SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HARRIS ACADEMY RAINHAM | 4-court | 32 x 18 | 576 | 1971 |  | Educational (in-house) | 60% | 40% | 73% | 9% | 18% |
| HARRIS ACADEMY RAINHAM | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HARROW LODGE LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2021 |  | Public | 100% |  | 76% | 9% | 16% |
| HAVERING SIXTH FORM COLLEGE | 4-court | 34.5 x 20 | 690 | 1999 |  | Educational (in-house) | 92% | 8% | 74% | 7% | 19% |
| HORNCHURCH HIGH SCHOOL | 4-court | 33 x 18 | 594 | 1965 |  | Educational (in-house) | 45% | 55% | 73% | 8% | 18% |
| HORNCHURCH HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| MARSHALLS PARK ACADEMY | 4-court | 33 x 18 | 594 | 1996 |  | Educational (in-house) | 100% |  | 72% | 10% | 18% |
| MARSHALLS PARK ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| NOAK HILL SPORTS COMPLEX | 3-court | 27 x 18 | 486 | 2017 |  | Public | 100% |  | 76% | 10% | 15% |
| REDDEN COURT SCHOOL | 3-court | 27 x 18 | 486 | 1969 | 2004 | Educational (in-house) | 57% | 43% | 73% | 8% | 18% |
| THE CAMPION SCHOOL | 4-court | 34.5 x 20 | 690 | 1996 | 2008 | Educational (in-house) | 71% | 29% | 75% | 9% | 16% |
| THE CAMPION SCHOOL | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| THE FRANCES BARDSLEY ACADEMY FOR GIRLS | 5-court | 40.6 x 21.35 | 867 | 2004 |  | Educational (in-house) | 100% |  | 70% | 9% | 20% |
| THE SANDERS SCHOOL | 4-court | 33 x 18 | 594 | 1995 |  | Educational (in-house) | 89% | 11% | 74% | 8% | 18% |
| Lewisham |  |  |  | 2006 |  |  | 100% | 0% | 49% | 14% | 37% |
| ADDEY AND STANHOPE SCHOOL | 4-court | 34.5 x 20 | 690 | 2004 |  | Educational (in-house) | 100% |  | 28% | 9% | 64% |
| BONUS PASTOR CATHOLIC COLLEGE | 4-court | 33 x 18 | 594 | 2012 |  | Educational (in-house) | 100% |  | 51% | 11% | 38% |
| DEPTFORD GREEN SCHOOL | 4-court | 33 x 18 | 594 | 2012 |  | Educational (in-house) | 100% |  | 38% | 13% | 50% |
| FOREST HILL SCHOOL SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2005 |  | Educational (3rd party) | 100% |  | 59% | 16% | 25% |
| FOREST HILL SCHOOL SPORTS CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HABERDASHERS' ASKE'S KNIGHTS ACADEMY | 3-court | 29.3 x 18 | 527 | 2007 |  | Educational (in-house) | 100% |  | 47% | 9% | 44% |
| LIONS CENTRE | 4-court | 33 x 18 | 594 | 1994 | 2005 | Public | 100% |  | 40% | 14% | 46% |
| PRENDERGAST SCHOOL | 4-court | 33 x 18 | 594 | 1996 | 2013 | Educational (in-house) | 100% |  | 44% | 12% | 44% |
| PRENDERGAST VALE | 4-court | 34.5 x 20 | 690 | 2011 |  | Educational (in-house) | 100% |  | 28% | 8% | 63% |
| SEDGEHILL ACADEMY | 4-court | 33 x 18 | 594 | 2009 |  | Educational (in-house) | 100% |  | 55% | 13% | 32% |
| ST DUNSTANS COLLEGE SPORTS CENTRE | 3-court | 27 x 18 | 486 | 1996 |  | Educational (3rd party) | 100% |  | 42% | 11% | 47% |
| SYDENHAM HIGH SCHOOL GDST | 4-court | 33 x 18 | 594 | 1993 |  | Educational (in-house) | 100% |  | 66% | 18% | 17% |
| SYDENHAM SCHOOL | 4-court | 33 x 18 | 594 | 2015 |  | Educational (in-house) | 100% |  | 65% | 18% | 17% |
| SYDENHAM SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| THE THUNDERDOME | 3-court | 56 x 18 | 1008 | 2017 |  | Other (sport club) | 100% |  | 49% | 18% | 33% |
| TIDEMILL ACADEMY | 4-court | 34.5 x 20 | 690 | 2012 |  | Educational (3rd party) | 100% |  | 25% | 8% | 66% |
| Newham |  |  |  | 1993 |  |  | 87% | 13% | 51% | 15% | 35% |
| ATHERTON LEISURE CENTRE | 4-court | 33 x 18 | 594 | 1935 | 2005 | Public | 100% |  | 48% | 12% | 40% |
| BRAMPTON MANOR ACADEMY | 6-court | 34.5 x 27 | 932 | 2005 |  | Educational (in-house) | 100% |  | 57% | 14% | 29% |
| BRAMPTON MANOR ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CARPENTERS & DOCKLANDS CENTRE | 3-court | 27 x 18 | 486 | 1972 | 2003 | Other (pay & play) | 82% | 18% | 43% | 16% | 41% |
| CHOBHAM ACADEMY | 4-court | 34.5 x 20 | 690 | 2012 |  | Educational (in-house) | 93% | 7% | 40% | 12% | 48% |
| CUMBERLAND COMMUNITY SCHOOL | 4-court | 33 x 18 | 594 | 2005 |  | Educational (3rd party) | 100% |  | 58% | 17% | 25% |
| CUMBERLAND COMMUNITY SCHOOL | Activity | 17 x 15 | 255 |  |  |  |  |  |  |  |  |
| EAST HAM LEISURE CENTRE | 4-court | 34.5 x 20 | 690 | 2001 |  | Public | 100% |  | 46% | 11% | 44% |
| EASTLEA COMMUNITY SCHOOL | 4-court | 34.5 x 20 | 690 | 2006 |  | Educational (in-house) | 100% |  | 46% | 15% | 39% |
| EASTLEA COMMUNITY SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| FOREST GATE COMMUNITY SCHOOL | 4-court | 34.5 x 20 | 690 | 2011 |  | Educational (in-house) | 100% |  | 43% | 10% | 47% |
| FOREST GATE COMMUNITY SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| KINGSFORD COMMUNITY SCHOOL | 6-court | 33 x 27 | 891 | 2002 |  | Educational (in-house) | 82% | 18% | 51% | 13% | 36% |
| KINGSFORD COMMUNITY SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| LISTER COMMUNITY SCHOOL | 5-court | 33 x 23 | 759 | 1984 | 2011 | Educational (in-house) | 100% |  | 50% | 13% | 38% |
| LISTER COMMUNITY SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| LISTER COMMUNITY SCHOOL | Activity | 20 x 18 | 360 |  |  |  |  |  |  |  |  |
| LITTLE ILFORD SCHOOL | 5-court | 33 x 23 | 759 | 1950 | 2015 | Educational (in-house) | 100% |  | 40% | 9% | 51% |
| LITTLE ILFORD SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| NEWHAM LEISURE CENTRE | 8-court | 37 x 34 | 1258 | 1990 |  | Public | 100% |  | 60% | 19% | 21% |
| NEWHAM LEISURE CENTRE | 6-court | 34.5 x 27 | 932 |  |  |  |  |  |  |  |  |
| NEWHAM SIXTH FORM COLLEGE | 6-court | 33 x 27 | 891 | 1975 | 1995 | Educational (in-house) | 57% | 43% | 58% | 17% | 25% |
| OASIS ACADEMY SILVERTOWN | 4-court | 34 x 18 | 612 | 2022 |  | Educational (in-house) | 100% |  | 35% | 14% | 51% |
| ROKEBY SCHOOL | 6-court | 34.5 x 27 | 932 | 2010 |  | Educational (in-house) | 100% |  | 45% | 15% | 41% |
| ROYAL DOCKS ACADEMY | 4-court | 33 x 18 | 594 | 1999 |  | Educational (in-house) | 93% | 7% | 54% | 17% | 29% |
| ROYAL DOCKS ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| SARAH BONNELL SCHOOL | 5-court | 33 x 23 | 759 | 1985 | 2004 | Educational (3rd party) | 100% |  | 47% | 13% | 41% |
| SCHOOL 21 | 4-court | 34.5 x 20 | 690 | 2000 |  | Educational (in-house) | 100% |  | 36% | 14% | 50% |
| SPORTSDOCK | 10-court | 40 x 38 | 1520 | 2012 |  | Educational (in-house) | 43% | 57% | 67% | 19% | 14% |
| SPORTSDOCK | 9-court | 40 x 38 | 1520 |  |  |  |  |  |  |  |  |
| ST ANGELAS URSULINE SCHOOL | 4-court | 34.5 x 20 | 690 | 2006 |  | Educational (in-house) | 100% |  | 33% | 8% | 59% |
| ST BONAVENTURE'S SCHOOL | 4-court | 34.5 x 20 | 690 | 1965 |  | Educational (in-house) | 100% |  | 45% | 11% | 44% |
| Redbridge |  |  |  | 1987 |  |  | 85% | 15% | 68% | 10% | 22% |
| ARK ISAAC NEWTON ACADEMY | 4-court | 34.5 x 20 | 690 | 2012 |  | Educational (in-house) | 100% |  | 56% | 10% | 34% |
| ARK ISAAC NEWTON ACADEMY | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| ATAM ACADEMY | 4-court | 33 x 18 | 594 | 2018 |  | Educational (in-house) | 100% |  | 66% | 10% | 24% |
| BANCROFT'S SCHOOL | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 100% |  | 74% | 9% | 17% |
| BEAL HIGH SCHOOL | 4-court | 33 x 18 | 594 | 1999 |  | Educational (in-house) | 71% | 29% | 71% | 11% | 18% |
| BEAL HIGH SCHOOL | 3-court | 27 x 18 | 486 |  |  |  |  |  |  |  |  |
| BEAL HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CATERHAM HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 1997 |  | Educational (in-house) | 59% | 41% | 71% | 9% | 19% |
| CATERHAM HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CATERHAM HIGH SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| FOREST ACADEMY | 3-court | 27 x 18 | 486 | 1960 |  | Educational (in-house) | 61% | 39% | 41% | 5% | 53% |
| FRENFORD YOUTH CLUB | 4-court | 34.5 x 20 | 690 | 2010 |  | Other (sport club) | 100% |  | 58% | 12% | 30% |
| ILFORD COUNTY HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 2006 |  | Educational (in-house) | 73% | 27% | 73% | 10% | 17% |
| KANTOR KING SOLOMON HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 1993 |  | Educational (in-house) | 58% | 42% | 76% | 10% | 13% |
| KANTOR KING SOLOMON HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| LOXFORD LEISURE CENTRE | 4-court | 33 x 18 | 594 | 1965 | 2021 | Public | 100% |  | 56% | 10% | 34% |
| MAYFIELD SCHOOL (DAGENHAM) | 4-court | 33 x 18 | 594 | 1930 |  | Educational (in-house) | 100% |  | 58% | 9% | 32% |
| MAYFIELD SCHOOL (DAGENHAM) | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| OAKS PARK HIGH SCHOOL | 4-court | 33 x 18 | 594 | 2002 |  | Educational (in-house) | 93% | 7% | 61% | 9% | 30% |
| PALMER CATHOLIC ACADEMY | 3-court | 27 x 18 | 486 | 1965 |  | Educational (in-house) | 100% |  | 49% | 8% | 43% |
| PALMER CATHOLIC ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| REDBRIDGE SPORTS & LEISURE | 12-court | 60 x 34.5 | 2070 | 2012 |  | Other (pay & play) | 100% |  | 81% | 11% | 8% |
| REDBRIDGE SPORTS & LEISURE | 5-court | 40.6 x 21.35 | 867 |  |  |  |  |  |  |  |  |
| SEVEN KINGS HIGH SCHOOL | 3-court | 27 x 18 | 486 | 1984 |  | Educational (in-house) | 100% |  | 51% | 9% | 40% |
| SEVEN KINGS HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| ST AUBYNS SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1997 |  | Educational (in-house) | 100% |  | 73% | 11% | 16% |
| TRINITY CATHOLIC HIGH SCHOOL (UPPER SITE) | 3-court | 27 x 18 | 486 | 1975 | 2002 | Educational (in-house) | 55% | 45% | 66% | 8% | 26% |
| WANSTEAD LEISURE CENTRE | 5-court | 40.6 x 21.35 | 867 | 1974 | 1998 | Commercial | 32% | 68% | 52% | 7% | 41% |
| WANSTEAD LEISURE CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| WANSTEAD YOUTH CENTRE | 4-court | 34.5 x 20 | 690 | 1945 | 2010 | Public | 53% | 47% | 63% | 11% | 26% |
| WOODBRIDGE HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 1996 |  | Educational (in-house) | 87% | 13% | 65% | 9% | 26% |
| WOODBRIDGE HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| Tower Hamlets |  |  |  | 1997 |  |  | 98% | 2% | 33% | 16% | 51% |
| BOBBY MOORE ACADEMY | 4-court | 33 x 18 | 594 | 2018 |  | Educational (in-house) | 98% | 2% | 40% | 18% | 42% |
| BOW SCHOOL | 4-court | 34.5 x 20 | 690 | 2014 |  | Educational (in-house) | 100% |  | 28% | 12% | 60% |
| GEORGE GREEN'S SCHOOL & SPORTS CENTRE | 3-court | 27 x 18 | 486 | 1975 | 2006 | Educational (in-house) | 83% | 17% | 42% | 24% | 35% |
| GEORGE GREEN'S SCHOOL & SPORTS CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| GEORGE GREEN'S SCHOOL & SPORTS CENTRE | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| HAILEYBURY YOUTH CENTRE | 3-court | 27 x 18 | 486 | 2015 |  | Other (pay & play) | 100% |  | 31% | 12% | 58% |
| IN2SPORTS | 4-court | 33 x 18 | 594 | 2021 |  | Other (pay & play) | 100% |  | 30% | 17% | 53% |
| JOHN ORWELL SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1980 |  | Public | 100% |  | 35% | 14% | 51% |
| MILE END PARK LEISURE CENTRE AND STADIUM | 4-court | 33 x 18 | 594 | 2006 | 2014 | Public | 100% |  | 27% | 11% | 62% |
| MULBERRY ACADEMY SHOREDITCH | 6-court | 34.5 x 27 | 932 | 1999 |  | Educational (in-house) | 100% |  | 32% | 15% | 53% |
| MULBERRY SPORTS & LEISURE CENTRE | 4-court | 34.5 x 20 | 690 | 2003 | 2011 | Other (pay & play) | 100% |  | 27% | 12% | 60% |
| POPLAR BATHS | 4-court | 34.5 x 20 | 690 | 1933 | 2016 | Public | 100% |  | 26% | 12% | 61% |
| Q MOTION HEALTH & FITNESS | 3-court | 27 x 18 | 486 | 2008 |  | Educational (in-house) | 100% |  | 31% | 13% | 56% |
| STEPNEY ALL SAINTS SCHOOL | 4-court | 33 x 18 | 594 | 1995 |  | Educational (in-house) | 100% |  | 32% | 13% | 55% |
| SWANLEA SCHOOL | 4-court | 34.5 x 20 | 690 | 1993 |  | Educational (in-house) | 100% |  | 30% | 14% | 56% |
| SWANLEA SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| WHITECHAPEL SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1998 | 2021 | Public | 100% |  | 30% | 14% | 56% |
| Barnet |  |  |  | 1999 |  |  | 95% | 5% | 68% | 12% | 21% |
| ARCHER ACADEMY | 3-court | 27 x 18 | 486 | 2015 |  | Educational (in-house) | 100% |  | 59% | 12% | 30% |
| ASHMOLE ACADEMY | 4-court | 33 x 18 | 594 | 2004 |  | Educational (in-house) | 100% |  | 72% | 11% | 17% |
| ASHMOLE ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BARNET BURNT OAK LEISURE CENTRE | 4-court | 33 x 22 | 726 | 2003 |  | Public | 100% |  | 60% | 10% | 29% |
| BARNET COPTHALL LEISURE CENTRE | 4-court | 40 x 18 | 720 | 2019 |  | Public | 100% |  | 74% | 13% | 13% |
| BISHOP DOUGLASS SCHOOL | 4-court | 34.5 x 20 | 690 | 2005 |  | Educational (in-house) | 100% |  | 66% | 13% | 21% |
| BROADFIELDS PRIMARY SCHOOL | 4-court | 33 x 18 | 594 | 1952 |  | Educational (in-house) | 82% | 18% | 54% | 7% | 39% |
| CHRIST'S COLLEGE FINCHLEY | 3-court | 27 x 18 | 486 | 1991 |  | Educational (in-house) | 100% |  | 70% | 14% | 16% |
| CHRIST'S COLLEGE FINCHLEY | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| COPTHALL SCHOOL | 4-court | 34.5 x 20 | 690 | 1995 |  | Educational (in-house) | 100% |  | 74% | 13% | 14% |
| DAVID LLOYD (FINCHLEY) | 4-court | 34.5 x 20 | 690 | 2007 |  | Commercial | 100% |  | 72% | 12% | 16% |
| EAST BARNET SCHOOL | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (in-house) | 100% |  | 71% | 10% | 19% |
| HASMONEAN BOYS SCHOOL | 3-court | 27 x 18 | 486 | 1982 |  | Educational (in-house) | 100% |  | 62% | 12% | 26% |
| HENDON LEISURE CENTRE | 4-court | 42 x 18 | 756 | 1995 |  | Public | 100% |  | 66% | 17% | 17% |
| HENDON LEISURE CENTRE | 3-court | 30 x 20 | 600 |  |  |  |  |  |  |  |  |
| HENDON LEISURE CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HENDON SCHOOL | 3-court | 27 x 18 | 486 | 1970 | 2009 | Educational (in-house) | 100% |  | 56% | 12% | 32% |
| HENDON SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| JEWISH COMMUNITY SECONDARY SCHOOL | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (in-house) | 72% | 28% | 54% | 8% | 39% |
| LONDON ACADEMY | 4-court | 34.5 x 20 | 690 | 2006 |  | Educational (in-house) | 100% |  | 81% | 11% | 9% |
| MILL HILL SCHOOL SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1983 | 1999 | Educational (in-house) | 87% | 13% | 80% | 12% | 8% |
| MILL HILL SCHOOL SPORTS CENTRE | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| ORION PRIMARY SCHOOL | 3-court | 33 x 18 | 594 | 2015 |  | Educational (in-house) | 100% |  | 69% | 11% | 20% |
| POWERLEAGUE (FINCHLEY) | 4-court | 33 x 18 | 594 | 1997 | 2005 | Commercial | 100% |  | 75% | 12% | 14% |
| QUEEN ELIZABETH SPORTS CENTRE | 6-court | 51 x 18 | 918 | 1975 | 2009 | Educational (in-house) | 86% | 14% | 67% | 9% | 24% |
| QUEEN ELIZABETH'S SCHOOL | 4-court | 33 x 18 | 594 | 2009 |  | Educational (in-house) | 81% | 19% | 75% | 9% | 15% |
| QUEEN ELIZABETH'S SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| SARACENS HIGH SCHOOL | 4-court | 33 x 18 | 594 | 2021 |  | Educational (in-house) | 100% |  | 61% | 11% | 28% |
| ST JAMES CATHOLIC HIGH SCHOOL (EDGWARE) | 3-court | 27 x 18 | 486 | 1999 |  | Educational (in-house) | 100% |  | 56% | 11% | 33% |
| ST JAMES CATHOLIC HIGH SCHOOL (EDGWARE) | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| TOTTERIDGE ACADEMY | 3-court | 27 x 18 | 486 | 1990 |  | Educational (in-house) | 67% | 33% | 81% | 11% | 8% |
| TOTTERIDGE ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| UNITAS YOUTH ZONE | 4-court | 33 x 18 | 594 | 2019 |  | Other (pay & play) | 100% |  | 55% | 10% | 35% |
| WOODHOUSE COLLEGE | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 100% |  | 58% | 11% | 31% |
| Enfield |  |  |  | 1993 |  |  | 94% | 6% | 66% | 11% | 23% |
| AIM NORTH LONDON | 6-court | 34.5 x 27 | 932 | 2008 |  | Educational (3rd party) | 100% |  | 59% | 12% | 29% |
| ARK JOHN KEATS ACADEMY | 4-court | 34.5 x 20 | 690 | 2015 |  | Educational (in-house) | 100% |  | 52% | 10% | 38% |
| AYLWARD ACADEMY | 4-court | 34.5 x 20 | 690 | 1991 |  | Educational (in-house) | 100% |  | 62% | 13% | 26% |
| AYLWARD ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CHACE COMMUNITY SCHOOL | 4-court | 34.5 x 20 | 690 | 1973 | 2015 | Educational (in-house) | 100% |  | 69% | 11% | 21% |
| CHACE COMMUNITY SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CONEL (ENFIELD CENTRE) | 4-court | 34.5 x 20 | 690 | 1970 |  | Educational (in-house) | 100% |  | 55% | 10% | 35% |
| DAVID LLOYD (ENFIELD) | 4-court | 34.5 x 20 | 690 | 2004 |  | Commercial | 60% | 40% | 81% | 8% | 11% |
| EDMONTON LEISURE CENTRE | 6-court | 33 x 27 | 891 | 2007 |  | Public | 100% |  | 47% | 11% | 41% |
| ENFIELD GRAMMAR SCHOOL | 3-court | 33 x 18 | 594 | 1970 |  | Educational (in house) | 70% | 30% | 63% | 10% | 27% |
| ENFIELD GRAMMAR SCHOOL | 3-court | 27 x 18 | 486 |  |  |  |  |  |  |  |  |
| HIGHLANDS SCHOOL | 4-court | 33 x 18 | 594 | 2000 |  | Educational (in-house) | 100% |  | 78% | 11% | 12% |
| HIGHLANDS SCHOOL | Activity | 20 x 15 | 300 |  |  |  |  |  |  |  |  |
| LAUREL PARK SCHOOL | 5-court | 40.6 x 21.35 | 867 | 1995 |  | Educational (in-house) | 100% |  | 62% | 11% | 27% |
| LAUREL PARK SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| LEA VALLEY ACADEMY | 4-court | 33 x 27 | 891 | 1990 | 2013 | Educational (in-house) | 100% |  | 73% | 12% | 14% |
| LEA VALLEY ACADEMY | Activity | 20 x 12 | 240 |  |  |  |  |  |  |  |  |
| OASIS ACADEMY HADLEY | 4-court | 33 x 18 | 594 | 2013 |  | Educational (in-house) | 100% |  | 53% | 10% | 37% |
| SOUTHBURY LEISURE CENTRE | 3-court | 34.5 x 20 | 690 | 2002 |  | Public | 100% |  | 67% | 12% | 22% |
| SOUTHGATE SCHOOL | 4-court | 33 x 18 | 594 | 1995 |  | Educational (in-house) | 88% | 12% | 78% | 11% | 12% |
| SOUTHGATE SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| SOUTHGATE SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| WINCHMORE SCHOOL | 4-court | 33 x 18 | 594 | 1965 | 2011 | Educational (in-house) | 100% |  | 66% | 11% | 23% |
| Haringey |  |  |  | 1994 |  |  | 94% | 6% | 50% | 15% | 35% |
| ALEXANDRA PARK SCHOOL | 4-court | 34.5 x 20 | 690 | 1980 |  | Educational (in-house) | 62% | 38% | 63% | 14% | 23% |
| ALEXANDRA PARK SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| BROADWATER FARM COMMUNITY CENTRE | 3-court | 27 x 18 | 486 | 1998 |  | Public | 100% |  | 34% | 14% | 52% |
| DUKE'S ALDRIDGE ACADEMY | 4-court | 34.5 x 20 | 690 | 2003 |  | Educational (in-house) | 81% | 19% | 47% | 13% | 39% |
| DUKE'S ALDRIDGE ACADEMY | 4-court | 30 x 20 | 600 |  |  |  |  |  |  |  |  |
| FORTISMERE SCHOOL | 4-court | 34.5 x 20 | 690 | 2004 |  | Educational (in-house) | 100% |  | 63% | 14% | 23% |
| FORTISMERE SCHOOL | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| GLADESMORE SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1986 | 2002 | Educational (in-house) | 100% |  | 39% | 16% | 46% |
| GREIG CITY ACADEMY | 4-court | 35 x 20 | 700 | 2002 |  | Educational (in-house) | 100% |  | 52% | 18% | 30% |
| GREIG CITY ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HARINGEY SIXTH FORM COLLEGE | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 89% | 11% | 49% | 12% | 39% |
| HEARTLANDS HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (in-house) | 100% |  | 51% | 16% | 33% |
| HIGHGATE SCHOOL (MALLINSON SPORTS CENTRE) | 4-court | 33 x 18 | 594 | 1989 |  | Educational (in-house) | 100% |  | 58% | 16% | 25% |
| HIGHGATE WOOD SCHOOL | 4-court | 33 x 18 | 594 | 1980 | 2004 | Educational (in-house) | 92% | 8% | 53% | 17% | 31% |
| HIGHGATE WOOD SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HORNSEY SCHOOL FOR GIRLS | 5-court | 40.6 x 21.35 | 867 | 2000 |  | Educational (in-house) | 100% |  | 51% | 20% | 30% |
| HORNSEY SCHOOL FOR GIRLS | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| MULBERRY ACADEMY WOODSIDE | 4-court | 34.5 x 20 | 690 | 2011 |  | Educational (in-house) | 100% |  | 52% | 15% | 32% |
| PARK VIEW SCHOOL | 4-court | 34.5 x 20 | 690 | 2000 | 2010 | Educational (in-house) | 100% |  | 41% | 18% | 41% |
| PARK VIEW SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| SELBY CENTRE | 4-court | 36 x 18 | 648 | 1975 | 2007 | Other (pay & play) | 95% | 5% | 53% | 14% | 33% |
| SELBY CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| ST THOMAS MORE CATHOLIC SCHOOL | 4-court | 34.5 x 20 | 690 | 1985 | 2013 | Educational (in-house) | 100% |  | 48% | 13% | 38% |
| ST THOMAS MORE CATHOLIC SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| TOTTENHAM COMMUNITY SPORTS CENTRE | 4-court | 34.5 x 20 | 690 | 1970 |  | Other (pay & play) | 99% | 1% | 48% | 15% | 37% |
| TOTTENHAM GREEN POOLS AND FITNESS | 6-court | 34 x 27 | 918 | 1991 | 2013 | Public | 100% |  | 41% | 17% | 42% |
| Waltham Forest |  |  |  | 2004 |  |  | 95% | 5% | 57% | 14% | 29% |
| BUXTON SCHOOL | 5-court | 34 x 27 | 918 | 2017 |  | Educational (in-house) | 100% |  | 36% | 9% | 55% |
| CHINGFORD FOUNDATION SCHOOL | 4-court | 33 x 18 | 594 | 2007 |  | Educational (in-house) | 100% |  | 43% | 6% | 51% |
| CHINGFORD FOUNDATION SCHOOL | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| FOREST SCHOOL (SYLVESTRIAN LEISURE CENTRE) | 4-court | 33 x 18 | 594 | 1976 | 2016 | Educational (in-house) | 67% | 33% | 66% | 13% | 21% |
| FOREST SCHOOL (SYLVESTRIAN LEISURE CENTRE) | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| FOREST SCHOOL (SYLVESTRIAN LEISURE CENTRE) | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| FREDERICK BREMER SCHOOL | 4-court | 34.5 x 20 | 690 | 2008 |  | Educational (in-house) | 100% |  | 64% | 11% | 25% |
| LEYTON SIXTH FORM COLLEGE | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (in-house) | 91% | 9% | 49% | 12% | 39% |
| LEYTON SPORTS GROUND | 4-court | 33 x 18 | 594 | 1973 | 2019 | Public | 100% |  | 50% | 14% | 36% |
| LEYTONSTONE SCHOOL | 4-court | 33 x 18 | 594 | 2006 |  | Educational (in-house) | 82% | 18% | 55% | 13% | 32% |
| PETER MAY SPORTS CENTRE | 6-court | 33 x 27 | 891 | 2000 |  | Other (sport club) | 90% | 10% | 72% | 13% | 16% |
| SCORE LEISURE CENTRE | 20-court | 90 x 33 | 2970 | 2024 |  | Public | 100% |  | 58% | 18% | 24% |
| SOUTH CHINGFORD FOUNDATION SCHOOL | 4-court | 34.5 x 20 | 690 | 1999 |  | Educational (in-house) | 98% | 2% | 64% | 12% | 24% |
| WALTHAM FOREST FEEL GOOD CENTRE | 4-court | 34.5 x 20 | 690 | 2016 |  | Public | 100% |  | 60% | 13% | 28% |
| WALTHAMSTOW ACADEMY | 4-court | 33 x 18 | 594 | 2008 |  | Educational (3rd party) | 100% |  | 63% | 13% | 23% |
| WALTHAMSTOW LEISURE CENTRE | 6-court | 33 x 27 | 891 | 1983 | 2013 | Public | 100% |  | 50% | 16% | 34% |
| WALTHAMSTOW LEISURE CENTRE | Activity | 20 x 10 | 200 |  |  |  |  |  |  |  |  |
| WALTHAMSTOW SCHOOL FOR GIRLS | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (3rd party) | 100% |  | 53% | 12% | 35% |
| WALTHAMSTOW SCHOOL FOR GIRLS | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| WOODFORD COUNTY HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 2007 | 2018 | Educational (in-house) | 100% |  | 70% | 10% | 20% |
| YMCA (WALTHAMSTOW) | 4-court | 35 x 20 | 700 | 2015 |  | Other (pay & play) | 100% |  | 53% | 12% | 35% |
| Bromley |  |  |  | 1992 |  |  | 80% | 20% | 73% | 13% | 13% |
| BICKLEY PARK SCHOOL | 3-court | 27 x 18 | 486 | 2007 |  | Educational (in-house) | 100% |  | 67% | 10% | 23% |
| BISHOP JUSTUS C OF E SCHOOL | 4-court | 34.5 x 20 | 690 | 2005 |  | Educational (in-house) | 97% | 3% | 77% | 9% | 14% |
| BROMLEY HIGH SCHOOL | 4-court | 36 x 20 | 720 | 1999 |  | Educational (in-house) | 57% | 43% | 73% | 9% | 18% |
| BROMLEY HIGH SCHOOL | Activity | 18 x 12 | 216 |  |  |  |  |  |  |  |  |
| BULLERS WOOD SCHOOL | 4-court | 34.5 x 20 | 690 | 1999 |  | Educational (in-house) | 100% |  | 75% | 11% | 13% |
| CHISLEHURST SCHOOL FOR GIRLS | 3-court | 27 x 18 | 486 | 1993 |  | Educational (in-house) | 59% | 41% | 82% | 12% | 6% |
| CHISLEHURST SCHOOL FOR GIRLS | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| COOPERS SCHOOL | 4-court | 34.5 x 20 | 690 | 1999 |  | Educational (in-house) | 89% | 11% | 83% | 11% | 5% |
| COOPERS SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| COOPERS SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CRYSTAL PALACE NATIONAL SPORTS CENTRE | 6-court | 46 x 29.18 | 1342 | 1964 | 2000 | Other (pay & play) | 83% | 17% | 67% | 18% | 15% |
| CRYSTAL PALACE NATIONAL SPORTS CENTRE | 4-court | 32 x 18 | 576 |  |  |  |  |  |  |  |  |
| CRYSTAL PALACE NATIONAL SPORTS CENTRE | Activity | 17.37 x 10.36 | 180 |  |  |  |  |  |  |  |  |
| CRYSTAL PALACE NATIONAL SPORTS CENTRE | Activity | 17.37 x 10.36 | 180 |  |  |  |  |  |  |  |  |
| DARRICK WOOD SPORTS CENTRE | 3-court | 27 x 18 | 486 | 1989 | 2018 | Educational (in-house) | 66% | 34% | 82% | 8% | 10% |
| DARRICK WOOD SPORTS CENTRE | Activity | 19 x 17 | 323 |  |  |  |  |  |  |  |  |
| DARWIN LEISURE CENTRE | 4-court | 33 x 20 | 660 | 1995 |  | Educational (in-house) | 40% | 60% | 87% | 6% | 7% |
| DARWIN LEISURE CENTRE | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| EDEN PARK HIGH SCHOOL | 4-court | 33 x 18.5 | 611 | 2019 |  | Educational (in-house) | 100% |  | 70% | 13% | 17% |
| ELTHAM COLLEGE | 3-court | 27 x 18 | 486 | 1970 |  | Educational (in-house) | 100% |  | 62% | 12% | 26% |
| ERIC LIDDELL SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1996 | 2023 | Educational (3rd party) | 100% |  | 63% | 12% | 25% |
| FARRINGTONS SCHOOL | 4-court | 34.5 x 20 | 690 | 1990 |  | Educational (in-house) | 82% | 18% | 83% | 12% | 5% |
| HARRIS ACADEMY BECKENHAM | 8-court | 39 x 33 | 1287 | 1968 | 2006 | Educational (in-house) | 71% | 29% | 75% | 13% | 12% |
| HARRIS ACADEMY ORPINGTON | 3-court | 27 x 18 | 486 | 1992 | 2004 | Educational (in-house) | 77% | 23% | 79% | 8% | 13% |
| HARRIS ACADEMY ORPINGTON | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HAYES SCHOOL | 3-court | 27 x 18 | 486 | 1985 |  | Educational (in-house) | 66% | 34% | 81% | 9% | 10% |
| HAYES SCHOOL | Activity | 17 x 10 | 170 |  |  |  |  |  |  |  |  |
| LANGLEY PARK SCHOOL FOR BOYS | 3-court | 27 x 18 | 486 | 1975 |  | Educational (in-house) | 50% | 50% | 77% | 12% | 11% |
| LANGLEY PARK SCHOOL FOR GIRLS | 4-court | 33 x 18 | 594 | 1993 |  | Educational (in-house) | 72% | 28% | 76% | 12% | 13% |
| LANGLEY PARK SCHOOL FOR GIRLS | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| RAVENSBOURNE SCHOOL | 3-court | 27 x 18 | 486 | 1994 | 2003 | Educational (in-house) | 100% |  | 61% | 9% | 30% |
| THE COUNTY GROUND (KENT) | 4-court | 34.5 x 20 | 690 | 2014 |  | Other (sport club) | 100% |  | 69% | 15% | 16% |
| THE PARKLANGLEY CLUB | 4-court | 40 x 18 | 720 | 2002 |  | Other (sport club) | 100% |  | 77% | 12% | 10% |
| WALNUTS LEISURE CENTRE | 5-court | 40.6 x 21.35 | 867 | 1980 | 2010 | Public | 93% | 7% | 82% | 8% | 10% |
| WALNUTS LEISURE CENTRE | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| Croydon |  |  |  | 2002 |  |  | 90% | 10% | 67% | 13% | 20% |
| 3D HEALTH & FITNESS (SOUTH NORWOOD) | 4-court | 33 x 18 | 594 | 2016 |  | Educational (3rd party) | 100% |  | 52% | 13% | 35% |
| COULSDON SIXTH FORM COLLEGE | 4-court | 34.5 x 20 | 690 | 2008 |  | Educational (in-house) | 83% | 17% | 81% | 7% | 13% |
| CROYDON SPORTS CLUB | 4-court | 34.5 x 20 | 690 | 2000 |  | Educational (3rd party) | 100% |  | 82% | 11% | 8% |
| CROYDON SPORTS CLUB | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HARRIS ACADEMY SOUTH NORWOOD | 4-court | 33 x 18 | 594 | 2007 |  | Educational (in house) | 100% |  | 44% | 11% | 45% |
| HARRIS CITY ACADEMY (CRYSTAL PALACE) | 4-court | 33 x 18 | 594 | 1990 | 2010 | Educational (in house) | 100% |  | 64% | 17% | 20% |
| HARRIS CITY ACADEMY (CRYSTAL PALACE) | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| JOHN RUSKIN COLLEGE | 3-court | 27 x 18 | 486 | 2005 |  | Educational (in-house) | 83% | 17% | 77% | 12% | 11% |
| MONKS HILL SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2015 |  | Educational (3rd party) | 100% |  | 75% | 11% | 14% |
| NEW ADDINGTON LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2019 |  | Public | 100% |  | 70% | 12% | 19% |
| OASIS ACADEMY COULSDON | 4-court | 34.5 x 20 | 690 | 2011 |  | Educational (in-house) | 74% | 26% | 82% | 7% | 11% |
| OASIS ACADEMY COULSDON | 3-court | 27 x 18 | 486 |  |  |  |  |  |  |  |  |
| OASIS ACADEMY SHIRLEY PARK | 4-court | 33 x 18 | 594 | 2006 | 2014 | Educational (in-house) | 100% |  | 59% | 14% | 27% |
| ORCHARD PARK HIGH SCHOOL | 3-court | 27 x 18 | 486 | 1978 | 2006 | Educational (in-house) | 77% | 23% | 69% | 12% | 20% |
| ORCHARD PARK HIGH SCHOOL | Activity | 20 x 18 | 360 |  |  |  |  |  |  |  |  |
| ROYAL RUSSELL SCHOOL | 6-court | 34.5 x 27 | 932 | 1995 |  | Educational (in-house) | 53% | 47% | 81% | 17% | 2% |
| ROYAL RUSSELL SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| SOUTH CROYDON SPORTS CLUB | 3-court | 27 x 18 | 486 | 1995 |  | Other (sport club) | 100% |  | 62% | 15% | 24% |
| ST JOSEPHS COLLEGE | 4-court | 33 x 18 | 594 | 2003 |  | Educational (in-house) | 100% |  | 67% | 17% | 16% |
| ST JOSEPHS COLLEGE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| THE ARCHBISHOP LANFRANC ACADEMY | 3-court | 27 x 18 | 486 | 1956 | 2005 | Educational (in-house) | 85% | 15% | 56% | 12% | 32% |
| THE ARCHBISHOP LANFRANC ACADEMY | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| THOMAS MORE CATHOLIC SCHOOL | 4-court | 34.5 x 20 | 690 | 2004 |  | Educational (in-house) | 44% | 56% | 64% | 9% | 27% |
| THORNTON HEATH LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2004 |  | Public | 100% |  | 39% | 9% | 52% |
| TRINITY SCHOOL (TRINITY SPORTS CLUB) | 5-court | 40.6 x 21.35 | 867 | 1994 |  | Educational (in-house) | 100% |  | 71% | 17% | 12% |
| TRINITY SCHOOL (TRINITY SPORTS CLUB) | 4-court | 34.5 x 20 | 690 |  |  |  |  |  |  |  |  |
| WADDON LEISURE CENTRE | 4-court | 33 x 18 | 594 | 2013 |  | Public | 100% |  | 63% | 16% | 21% |
| WHITGIFT SCHOOL | 7-court | 44 x 28 | 1232 | 2005 |  | Educational (in house) | 100% |  | 58% | 14% | 28% |
| WOODCOTE HIGH SCHOOL | 4-court | 33 x 18 | 594 | 2010 |  | Educational (in-house) | 83% | 17% | 79% | 8% | 13% |
| WOODCOTE HIGH SCHOOL | 3-court | 27 x 18 | 486 |  |  |  |  |  |  |  |  |
| WOODCOTE HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| Kingston upon Thames |  |  |  |  |  |  | 80% | 20% | 71% | 9% | 20% |
| ARENA SPORTS | 6-court | 34.5 x 27.1 | 935 | 2003 |  | Educational (in-house) | 79% | 21% | 53% | 8% | 39% |
| CHESSINGTON SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1992 | 2007 | Educational (3rd party) | 100% |  | 82% | 7% | 12% |
| COOMBE BOYS SCHOOL | 4-court | 33 x 18 | 594 | 1976 | 2003 | Educational (in-house) | 65% | 35% | 69% | 8% | 23% |
| COOMBE GIRLS' SCHOOL | 4-court | 33 x 18 | 594 | 2005 |  | Educational (in-house) | 89% | 11% | 71% | 10% | 19% |
| COOMBE GIRLS' SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| KINGS CENTRE | 4-court | 34.5 x 20 | 690 | 1997 |  | Other (pay & play) | 100% |  | 80% | 7% | 13% |
| RICHARD CHALLONER SCHOOL | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 89% | 11% | 68% | 6% | 26% |
| TERRITORIAL ARMY HALL | 3-court | 27 x 18 | 486 | 1936 |  | Other (sport club) | 54% | 46% | 55% | 9% | 36% |
| THE HOLY CROSS SCHOOL | 4-court | 34.5 x 20 | 690 | 2007 |  | Educational (in-house) | 100% |  | 60% | 7% | 33% |
| THE KINGSTON ACADEMY | 4-court | 33 x 18 | 594 | 2018 |  | Educational (in-house) | 64% | 36% | 71% | 9% | 19% |
| THE TIFFIN GIRLS' SCHOOL | 4-court | 35 x 22 | 770 | 2022 |  | Educational (in-house) | 58% | 42% | 73% | 9% | 18% |
| THE TIFFIN GIRLS' SCHOOL | Activity | 21 x 12 | 252 |  |  |  |  |  |  |  |  |
| THE TIFFIN GIRLS' SCHOOL | Activity | 21 x 12 | 252 |  |  |  |  |  |  |  |  |
| TIFFIN SCHOOL SPORTS CENTRE | 6-court | 34.5 x 27 | 932 | 1995 | 2011 | Educational (in-house) | 100% |  | 54% | 9% | 38% |
| TOLWORTH RECREATION CENTRE | 5-court | 33 x 26 | 858 | 1979 | 2004 | Other (pay & play) | 80% | 20% | 78% | 10% | 13% |
| TOLWORTH RECREATION CENTRE | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| TOLWORTH RECREATION CENTRE | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| Merton |  |  |  |  |  |  | 90% | 10% | 67% | 13% | 20% |
| CANONS LEISURE CENTRE (MITCHAM) | 6-court | 31 x 33 | 1023 | 1983 |  | Public | 100% |  | 64% | 13% | 23% |
| HARRIS ACADEMY MERTON | 4-court | 33 x 18 | 594 | 2004 |  | Educational (in house) | 100% |  | 62% | 12% | 26% |
| HARRIS ACADEMY MORDEN | 4-court | 34.5 x 20 | 690 | 2003 |  | Educational (in-house) | 100% |  | 67% | 11% | 22% |
| HARRIS ACADEMY MORDEN | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| RAYNES PARK HIGH SCHOOL | 4-court | 32 x 18 | 576 | 2004 | 2007 | Educational (in-house) | 100% |  | 68% | 8% | 24% |
| RICARDS LODGE HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 2004 |  | Educational (in-house) | 100% |  | 68% | 16% | 16% |
| RICARDS LODGE HIGH SCHOOL | Activity | 20 x 12 | 240 |  |  |  |  |  |  |  |  |
| RUTLISH SCHOOL | 4-court | 33 x 18 | 594 | 2004 | 2020 | Educational (in-house) | 100% |  | 66% | 12% | 22% |
| RUTLISH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| ST MARKS ACADEMY | 4-court | 34.5 x 20 | 690 | 2004 |  | Educational (in-house) | 100% |  | 58% | 12% | 30% |
| THE KING'S CLUB | 6-court | 36 x 25.2 | 907 | 2019 |  | Educational (in-house) | 94% | 6% | 76% | 14% | 10% |
| THE KING'S CLUB | 4-court | 35 x 22 | 770 |  |  |  |  |  |  |  |  |
| URSULINE HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 2003 |  | Educational (in-house) | 81% | 19% | 68% | 11% | 20% |
| WIMBLEDON COLLEGE | 4-court | 34.5 x 20 | 690 | 2000 |  | Educational (in-house) | 76% | 24% | 75% | 14% | 11% |
| WIMBLEDON HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 1996 |  | Educational (in-house) | 66% | 34% | 57% | 12% | 31% |
| WIMBLEDON RACQUETS & FITNESS CLUB | 4-court | 34.5 x 20 | 690 | 1985 |  | Other (sport club) | 58% | 42% | 64% | 12% | 24% |
| WIMBLEDON RACQUETS & FITNESS CLUB | 3-court | 27 x 18 | 486 |  |  |  |  |  |  |  |  |
| Richmond upon Thames |  |  |  |  |  |  | 88% | 12% | 69% | 10% | 21% |
| CHRISTS SCHOOL | 4-court | 34.5 x 20 | 690 | 2004 |  | Educational (in-house) | 100% |  | 58% | 10% | 32% |
| GERMAN SCHOOL LONDON | 5-court | 33 x 24 | 792 | 1975 |  | Educational (in-house) | 28% | 72% | 68% | 11% | 21% |
| GREY COURT SCHOOL | 4-court | 34.5 x 20 | 690 | 1975 | 2019 | Educational (in-house) | 52% | 48% | 65% | 8% | 27% |
| GREY COURT SCHOOL | Activity | 20 x 14 | 280 |  |  |  |  |  |  |  |  |
| HAMPTON SPORT AND FITNESS CENTRE | 4-court | 34.5 x 20 | 690 | 1999 | 2016 | Educational (3rd party) | 100% |  | 75% | 9% | 16% |
| HAMPTON SPORT AND FITNESS CENTRE | Activity | 19 x 17 | 323 |  |  |  |  |  |  |  |  |
| HAMPTON YOUTH PROJECT | 3-court | 27 x 18 | 486 | 1970 |  | Public | 62% | 38% | 66% | 8% | 26% |
| ORLEANS PARK SCHOOL | 4-court | 34.5 x 20 | 690 | 1973 |  | Educational (in-house) | 86% | 14% | 59% | 9% | 32% |
| RICHMOND UPON THAMES COLLEGE | 4-court | 33 x 19 | 627 | 1985 | 2006 | Educational (in-house) | 66% | 34% | 60% | 8% | 33% |
| SHENE SPORTS & FITNESS CENTRE | 4-court | 33 x 18 | 594 | 1985 | 2007 | Educational (3rd party) | 100% |  | 65% | 11% | 24% |
| ST MARYS UNIVERSITY | 6-court | 35 x 27 | 945 | 1995 |  | Educational (in-house) | 82% | 18% | 72% | 9% | 18% |
| ST MARYS UNIVERSITY | 6-court | 35 x 27 | 945 |  |  |  |  |  |  |  |  |
| ST MARYS UNIVERSITY | Activity | 24 x 12 | 288 |  |  |  |  |  |  |  |  |
| ST PAULS SCHOOL | 6-court | 34.5 x 27 | 932 | 1968 | 2005 | Educational (in-house) | 100% | 0% | 62% | 15% | 23% |
| ST PAULS SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| TEDDINGTON SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2010 | 2014 | Educational (3rd party) | 100% | 0% | 76% | 10% | 14% |
| WALDEGRAVE SCHOOL | 3-court | 27 x 18 | 486 | 2014 |  | Educational (in-house) | 100% |  | 72% | 9% | 19% |
| WALDEGRAVE SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| WHITTON SPORTS AND FITNESS CENTRE | 4-court | 36 x 18 | 648 | 2014 |  | Public | 100% |  | 75% | 10% | 15% |
| Sutton |  |  |  |  |  |  | 80% | 20% | 72% | 9% | 19% |
| CARSHALTON BOYS SPORTS COLLEGE | 4-court | 33 x 18 | 594 | 1990 |  | Educational (in-house) | 70% | 30% | 72% | 9% | 18% |
| CARSHALTON HIGH SCHOOL FOR GIRLS | 4-court | 34 x 20 | 680 | 1955 | 2007 | Educational (in-house) | 45% | 55% | 75% | 9% | 15% |
| CARSHALTON HIGH SCHOOL FOR GIRLS | Activity | 16.5 x 16 | 264 |  |  |  |  |  |  |  |  |
| CHEAM HIGH SCHOOL | 3-court | 33 x 18 | 594 | 1990 | 2023 | Educational (in-house) | 100% |  | 72% | 7% | 21% |
| CHEAM HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| GLENTHORNE HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 2002 |  | Educational (in-house) | 100% |  | 62% | 8% | 30% |
| GLENTHORNE HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| GREENSHAW HIGH SCHOOL | 4-court | 33 x 18 | 594 | 2004 |  | Educational (in-house) | 93% | 7% | 73% | 9% | 18% |
| GREENSHAW HIGH SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| OVERTON GRANGE SCHOOL | 4-court | 33 x 18 | 594 | 1996 |  | Educational (in-house) | 83% | 17% | 66% | 6% | 28% |
| OVERTON GRANGE SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| PULSE HEALTH & FITNESS CARSHALTON | 4-court | 34.5 x 20 | 690 | 2010 |  | Educational (3rd party) | 100% |  | 74% | 10% | 17% |
| SUTTON GRAMMAR SCHOOL | 5-court | 33 x 26 | 858 | 2005 |  | Educational (in-house) | 100% |  | 61% | 7% | 32% |
| SUTTON HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 1984 | 2011 | Educational (in-house) | 94% | 6% | 64% | 6% | 30% |
| THE JOHN FISHER SCHOOL | 4-court | 33 x 18 | 594 | 2004 | 2024 | Educational (in-house) | 79% | 21% | 81% | 10% | 9% |
| THE JOHN FISHER SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| THE PHOENIX CENTRE | 4-court | 33 x 18 | 594 | 2004 |  | Public | 100% |  | 78% | 11% | 12% |
| THE PHOENIX CENTRE | Activity | 17 x 10 | 170 |  |  |  |  |  |  |  |  |
| WALLINGTON HIGH SCHOOL FOR GIRLS | 4-court | 34.5 x 20 | 690 | 2011 |  | Educational (in-house) | 79% | 21% | 81% | 9% | 10% |
| WALLINGTON HIGH SCHOOL FOR GIRLS | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| WANDLE VALLEY ACADEMY | 3-court | 42 x 18 | 756 | 1989 |  | Educational (in-house) | 73% | 27% | 69% | 10% | 21% |
| WANDLE VALLEY ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| WESTCROFT LEISURE CENTRE | 8-court | 40 x 34.5 | 1380 | 1977 |  | Public | 61% | 39% | 73% | 10% | 17% |
| WESTCROFT LEISURE CENTRE | Activity | 21 x 11 | 231 |  |  |  |  |  |  |  |  |
| WILSONS SCHOOL | 4-court | 33 x 18 | 594 | 2005 |  | Educational (in-house) | 72% | 28% | 72% | 12% | 15% |
| WILSONS SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| Wandsworth |  |  |  |  |  |  | 100% | 0% | 58% | 15% | 27% |
| ARK PUTNEY ACADEMY | 4-court | 33 x 18 | 594 | 2015 |  | Educational (in-house) | 100% |  | 59% | 14% | 26% |
| ARK PUTNEY ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| ARK PUTNEY ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| ASPIRE CENTRE (SOUTHFIELDS ACADEMY) | 4-court | 34.5 x 20 | 690 | 2005 | 2009 | Educational (in-house) | 100% |  | 60% | 15% | 25% |
| ASPIRE CENTRE (SOUTHFIELDS ACADEMY) | 4-court | 34.5 x 20 | 690 |  |  |  |  |  |  |  |  |
| BATTERSEA SPORTS CENTRE | 4-court | 33 x 18 | 594 | 1996 | 2011 | Other (pay & play) | 100% |  | 45% | 13% | 43% |
| BURNTWOOD SCHOOL | 4-court | 33 x 18 | 594 | 1962 |  | Educational (in-house) | 100% |  | 65% | 17% | 19% |
| BURNTWOOD SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BURNTWOOD SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BURNTWOOD SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CHESTNUT GROVE ACADEMY | 3-court | 27 x 18 | 486 | 1990 | 2011 | Educational (in-house) | 100% |  | 44% | 12% | 44% |
| EMANUEL SCHOOL | 4-court | 34.5 x 20 | 690 | 2003 |  | Educational (in-house) | 100% |  | 50% | 13% | 37% |
| ERNEST BEVIN ACADEMY | 6-court | 33 x 27 | 891 | 2007 |  | Educational (in-house) | 100% |  | 62% | 16% | 21% |
| ERNEST BEVIN ACADEMY | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| IBSTOCK PLACE SCHOOL | 4-court | 34.5 x 20 | 690 | 2008 |  | Educational (in-house) | 100% |  | 61% | 13% | 26% |
| IBSTOCK PLACE SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| LATCHMERE LEISURE CENTRE | 5-court | 34.5 x 25 | 863 | 1985 | 2015 | Public | 100% |  | 46% | 14% | 40% |
| NEWTON PREPARATORY SCHOOL | 4-court | 34.5 x 20 | 690 | 2012 |  | Educational (in-house) | 100% |  | 47% | 18% | 35% |
| NEWTON PREPARATORY SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| ROEHAMPTON SPORT AND FITNESS CENTRE | 4-court | 34.5 x 20 | 690 | 1987 | 2012 | Public | 100% |  | 56% | 13% | 31% |
| SAINT CECILIA'S WANDSWORTH CHURCH OF ENGLAND SCHOOL | 3-court | 27 x 18 | 486 | 2003 |  | Educational (in-house) | 100% |  | 56% | 14% | 30% |
| TOOTING LEISURE CENTRE | 6-court | 34.5 x 27 | 932 | 1976 | 2015 | Public | 100% |  | 55% | 15% | 30% |
| WANDLE RECREATION CENTRE | 4-court | 34.5 x 20 | 690 | 1985 | 2007 | Public | 100% |  | 59% | 16% | 25% |
| Brent |  |  |  |  |  |  | 98% | 2% | 65% | 14% | 22% |
| ARK ACADEMY | 4-court | 33 x 18 | 594 | 2010 |  | Educational (in-house) | 100% |  | 48% | 11% | 41% |
| ARK ELVIN ACADEMY | 5-court | 40 x 22 | 880 | 2000 |  | Educational (in-house) | 100% |  | 59% | 13% | 27% |
| ARK ELVIN ACADEMY | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| ARK ELVIN ACADEMY | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| BRIDGE PARK COMMUNITY LEISURE CENTRE | 5-court | 40.6 x 21.35 | 867 | 1985 | 2004 | Public | 100% |  | 64% | 18% | 18% |
| CHARTERIS (KILBURN) SPORTS CENTRE | 3-court | 27 x 18 | 486 | 1980 |  | Other (pay & play) | 100% |  | 35% | 13% | 52% |
| CLAREMONT HIGH SCHOOL | 4-court | 33 x 20 | 660 | 2008 |  | Educational (in-house) | 100% |  | 76% | 12% | 12% |
| CLAREMONT HIGH SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CLAREMONT HIGH SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| HARRIS LOWE ACADEMY WILLESDEN | 6-court | 34.5 x 27 | 932 | 2003 |  | Educational (in-house) | 100% |  | 57% | 20% | 24% |
| JFS SCHOOL | 6-court | 35.64 x 25 | 891 | 2002 | 2010 | Educational (in-house) | 100% |  | 77% | 13% | 11% |
| JFS SCHOOL | 4-court | 33 x 18 | 594 |  |  |  |  |  |  |  |  |
| KINGSBURY HIGH SCHOOL (UPPER SITE) | 4-court | 34.5 x 20 | 690 | 1978 | 2018 | Educational (in-house) | 100% |  | 65% | 11% | 24% |
| OAKINGTON MANOR PRIMARY SCHOOL | 4-court | 34.5 x 20 | 690 | 2004 |  | Educational (in-house) | 100% |  | 61% | 14% | 25% |
| PRESTON MANOR SCHOOL | 4-court | 34.5 x 20 | 690 | 2008 |  | Educational (in-house) | 100% |  | 64% | 13% | 22% |
| PRESTON MANOR SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| PRESTON MANOR SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| QUEENS PARK COMMUNITY SCHOOL | 4-court | 33 x 18 | 594 | 1950 | 2019 | Educational (in-house) | 100% |  | 56% | 20% | 25% |
| UNIVERSITY OF WESTMINSTER (HARROW SPORTS HALL) | 4-court | 33 x 18 | 594 | 1970 |  | Educational (in-house) | 61% | 39% | 74% | 12% | 15% |
| VALE FARM SPORTS CENTRE | 5-court | 40.5 x 20 | 810 | 1979 | 2007 | Public | 100% |  | 73% | 12% | 15% |
| VALE FARM SPORTS CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| WILLESDEN SPORTS CENTRE | 4-court | 34.5 x 20 | 690 | 2006 |  | Public | 100% |  | 53% | 18% | 28% |
| Ealing |  |  |  |  |  |  | 100% | 0% | 66% | 12% | 22% |
| ALEC REED ACADEMY SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2005 |  | Educational (in-house) | 100% |  | 76% | 10% | 14% |
| ALEC REED ACADEMY SPORTS CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| DORMERS WELLS LEISURE CENTRE | 6-court | 33 x 27 | 891 | 1972 | 2011 | Public | 100% |  | 68% | 11% | 22% |
| ELTHORNE SPORTS CENTRE | 4-court | 34.5 x 20 | 690 | 1984 | 2005 | Educational (3rd party) | 100% |  | 62% | 13% | 24% |
| FEATHERSTONE SPORTS CENTRE (SOUTHALL) | 5-court | 40.6 x 21.35 | 867 | 1996 | 2020 | Educational (in-house) | 100% |  | 53% | 9% | 37% |
| GREENFORD SPORTS CENTRE | 4-court | 34.5 x 20 | 690 | 2008 |  | Educational (3rd party) | 100% |  | 68% | 10% | 22% |
| GREENFORD SPORTS CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| HANWELL COMMUNITY CENTRE | 4-court | 33 x 18 | 594 | 1938 | 2009 | Public | 100% |  | 69% | 13% | 18% |
| HANWELL COMMUNITY CENTRE | Activity | 24 x 16 | 384 |  |  |  |  |  |  |  |  |
| KAJIMA COMMUNITY AT BRENTSIDE HIGH SCHOOL | 4-court | 33.3 x 18.3 | 609 | 2003 |  | Educational (in-house) | 100% |  | 74% | 13% | 13% |
| NORTHOLT HIGH SPORTS CENTRE | 4-court | 34.5 x 20 | 690 | 2006 |  | Educational (in-house) | 100% |  | 71% | 8% | 21% |
| REYNOLDS SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2007 |  | Educational (3rd party) | 100% |  | 59% | 16% | 25% |
| ST BENEDICTS SCHOOL | 4-court | 31 x 18 | 558 | 1994 | 2009 | Educational (in-house) | 100% |  | 56% | 12% | 33% |
| THE ELLEN WILKINSON SCHOOL FOR GIRLS | 4-court | 34.5 x 20 | 690 | 2009 |  | Educational (in-house) | 100% |  | 61% | 15% | 23% |
| THE ELLEN WILKINSON SCHOOL FOR GIRLS | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| TWYFORD SPORTS CENTRE | 4-court | 33.1 x 18 | 596 | 1989 |  | Commercial | 100% |  | 70% | 15% | 16% |
| Hammersmith & Fulham |  |  |  |  |  |  | 100% | 0% | 46% | 13% | 42% |
| ARK BURLINGTON DANES ACADEMY | 4-court | 34.5 x 20 | 690 | 2002 |  | Educational (in-house) | 100% |  | 51% | 18% | 31% |
| HARBOUR CLUB (CHELSEA) | 3-court | 27 x 18 | 486 | 2015 |  | Commercial | 100% |  | 61% | 13% | 26% |
| LATYMER UPPER SCHOOL | 6-court | 33 x 27 | 891 | 1976 |  | Educational (in-house) | 100% |  | 38% | 11% | 52% |
| WEST LONDON COLLEGE (HAMMERSMITH CAMPUS) | 4-court | 33 x 18 | 594 | 2008 | 2014 | Educational (in-house) | 100% |  | 31% | 9% | 59% |
| Harrow |  |  |  |  |  |  | 95% | 5% | 73% | 10% | 18% |
| ASPIRE LEISURE CENTRE | 4-court | 34.5 x 20 | 690 | 1990 |  | Other (sport club) | 53% | 47% | 86% | 10% | 4% |
| AVANTI HOUSE SECONDARY SCHOOL | 4-court | 33 x 18 | 594 | 2018 |  | Educational (in-house) | 100% |  | 73% | 9% | 18% |
| BENTLEY WOOD HIGH SCHOOL FOR GIRLS | 4-court | 34.5 x 20 | 690 | 2015 | 2024 | Educational (in-house) | 100% |  | 81% | 9% | 10% |
| BENTLEY WOOD HIGH SCHOOL FOR GIRLS | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BENTLEY WOOD HIGH SCHOOL FOR GIRLS | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| CEDARS MYPLACE YOUTH & COMMUNITY CENTRE | 3-court | 27 x 18 | 486 | 2012 |  | Public | 100% |  | 71% | 8% | 21% |
| HARROW HIGH SCHOOL | 4-court | 33 x 18 | 594 | 2001 |  | Educational (in-house) | 100% |  | 61% | 10% | 29% |
| HARROW LEISURE CENTRE | 10-court | 40.6 x 42.7 | 1734 | 1977 |  | Public | 100% |  | 72% | 10% | 18% |
| HARROW LEISURE CENTRE | 5-court | 40.6 x 21.35 | 867 |  |  |  |  |  |  |  |  |
| HARROW SCHOOL SPORTS COMPLEX | 4-court | 33 x 18 | 594 | 1985 | 2008 | Educational (in-house) | 100% |  | 65% | 10% | 24% |
| NORTH LONDON COLLEGIATE SCHOOL | 4-court | 34.5 x 20 | 690 | 1993 |  | Educational (in-house) | 77% | 23% | 60% | 9% | 31% |
| NOWER HILL HIGH SCHOOL | 4-court | 33 x 18 | 594 | 1995 |  | Educational (in-house) | 100% |  | 76% | 9% | 15% |
| NOWER HILL HIGH SCHOOL | Activity | 18 x 17 | 306 |  |  |  |  |  |  |  |  |
| ST DOMINIC'S SIXTH FORM COLLEGE | 3-court | 27 x 18 | 486 | 2010 |  | Educational (in-house) | 100% |  | 63% | 10% | 27% |
| WHITEFRIARS COMMUNITY SCHOOL | 4-court | 34.5 x 20 | 690 | 2000 |  | Educational (in-house) | 100% |  | 61% | 8% | 31% |
| Hillingdon |  |  |  |  |  |  | 93% | 7% | 77% | 9% | 14% |
| BARNHILL COMMUNITY HIGH SCHOOL | 4-court | 34.5 x 20 | 690 | 1999 | 2008 | Educational (in-house) | 100% |  | 64% | 8% | 28% |
| BISHOP RAMSEY CE SCHOOL | 3-court | 27 x 18 | 486 | 1950 |  | Educational (in-house) | 100% |  | 68% | 6% | 26% |
| BOTWELL GREEN SPORTS AND LEISURE CENTRE | 4-court | 33 x 19 | 627 | 2010 |  | Public | 100% |  | 69% | 10% | 21% |
| BOTWELL GREEN SPORTS AND LEISURE CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| BRUNEL UNIVERSITY SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2005 |  | Educational (in-house) | 100% |  | 63% | 8% | 29% |
| BRUNEL UNIVERSITY SPORTS CENTRE | 4-court | 33 x 18 | 594 |  |  |  |  |  |  |  |  |
| DAVID LLOYD (NORTHWOOD) | 3-court | 27 x 18 | 486 | 1996 | 2007 | Commercial | 66% | 34% | 87% | 7% | 6% |
| HAREFIELD SCHOOL | 4-court | 33 x 18 | 594 | 2008 |  | Educational (in-house) | 53% | 47% | 84% | 7% | 9% |
| HARLINGTON SPORTS CENTRE | 4-court | 33 x 20 | 660 | 2023 |  | Educational (3rd party) | 100% |  | 77% | 10% | 13% |
| HILLINGDON SPORTS AND LEISURE COMPLEX | 4-court | 34.5 x 20 | 690 | 2010 |  | Public | 100% |  | 83% | 10% | 7% |
| HILLINGDON SPORTS AND LEISURE COMPLEX | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| HILLINGDON SPORTS AND LEISURE COMPLEX | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| NORTHWOOD COLLEGE FOR GIRLS | 4-court | 34.5 x 20 | 690 | 1993 |  | Educational (in-house) | 100% |  | 80% | 8% | 12% |
| PARK ACADEMY WEST LONDON | 4-court | 33 x 18 | 594 | 2005 |  | Educational (in-house) | 100% |  | 44% | 6% | 50% |
| QUEENSMEAD SPORTS CENTRE | 6-court | 34.5 x 27 | 932 | 1976 | 2004 | Educational (3rd party) | 100% |  | 80% | 9% | 12% |
| ROSEDALE COLLEGE | 5-court | 40.6 x 21.35 | 867 | 1970 | 2006 | Educational (in-house) | 100% |  | 70% | 9% | 21% |
| ROSEDALE COLLEGE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| ST HELENS SCHOOL SPORTS CENTRE | 4-court | 33 x 18 | 594 | 2006 |  | Educational (in-house) | 100% |  | 78% | 8% | 13% |
| SWAKELEYS SCHOOL | 4-court | 33 x 18 | 594 | 1986 | 2010 | Educational (in-house) | 80% | 20% | 82% | 10% | 8% |
| SWAKELEYS SCHOOL | 4-court | 33 x 18 | 594 |  |  |  |  |  |  |  |  |
| SWAKELEYS SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| SWAKELEYS SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| THE DOUAY MARTYRS SCHOOL | 3-court | 27 x 18 | 486 | 1996 |  | Educational (in-house) | 100% |  | 74% | 8% | 19% |
| UXBRIDGE COLLEGE | 6-court | 34.5 x 27 | 932 | 2010 |  | Educational (in-house) | 100% |  | 81% | 10% | 9% |
| Hounslow |  |  |  |  |  |  | 97% | 3% | 70% | 14% | 17% |
| BOLDER ACADEMY | 4-court | 33 x 18 | 594 | 2020 |  | Educational (in-house) | 100% |  | 79% | 15% | 6% |
| BRENTFORD FOUNTAIN LEISURE CENTRE | 8-court | 37 x 33 | 1221 | 1987 | 2009 | Public | 100% |  | 70% | 17% | 13% |
| BRENTFORD FOUNTAIN LEISURE CENTRE | Activity | 17 x 9 | 153 |  |  |  |  |  |  |  |  |
| CHISWICK SCHOOL | 5-court | 33 x 26 | 858 | 1984 |  | Educational (in-house) | 100% |  | 72% | 18% | 10% |
| CHISWICK SCHOOL | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| CRANFORD COMMUNITY COLLEGE SPORTS CENTRE | 5-court | 40.6 x 21.35 | 867 | 1975 | 2005 | Educational (3rd party) | 100% |  | 77% | 11% | 11% |
| CRANFORD COMMUNITY COLLEGE SPORTS CENTRE | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| DAVID LLOYD (HESTON) | 3-court | 27 x 18 | 486 | 1982 | 2003 | Commercial | 30% | 70% | 85% | 11% | 4% |
| GUMLEY HOUSE CONVENT SCHOOL | 4-court | 34.5 x 20 | 690 | 1970 |  | Educational (in-house) | 100% |  | 68% | 11% | 21% |
| GUNNERSBURY CATHOLIC SCHOOL | 4-court | 33 x 18 | 594 | 2020 |  | Educational (in-house) | 100% |  | 54% | 12% | 34% |
| GUNNERSBURY PARK SPORTS HUB | 5-court | 44 x 22 | 968 | 2020 |  | Public | 100% |  | 68% | 17% | 15% |
| HESTON SPORTS CENTRE | 3-court | 30 x 18 | 540 | 1986 | 2010 | Educational (3rd party) | 100% |  | 67% | 11% | 22% |
| ISLEWORTH AND SYON SCHOOL FOR BOYS | 4-court | 33 x 18 | 594 | 1965 | 2009 | Educational (in-house) | 98% | 2% | 73% | 12% | 15% |
| ISLEWORTH AND SYON SCHOOL FOR BOYS | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| KINGSLEY ACADEMY | 4-court | 34.5 x 20 | 690 | 2017 |  | Educational (3rd party) | 100% |  | 51% | 10% | 39% |
| KINGSLEY ACADEMY | Activity | 18 x 10 | 180 |  |  |  |  |  |  |  |  |
| LAMPTON SCHOOL | 4-court | 33 x 18 | 594 | 2005 |  | Educational (3rd party) | 100% |  | 63% | 11% | 26% |
| MIDDLESEX BADMINTON DOME | 3-court | 34.5 x 27 | 932 | 1980 | 2005 | Other (pay & play) | 100% |  | 65% | 10% | 25% |
| NISHKAM SCHOOL (WEST LONDON) | 4-court | 33 x 18 | 594 | 2018 |  | Educational (in-house) | 100% |  | 80% | 15% | 5% |
| OSTERLEY SPORTS AND ATHLETICS CENTRE | 8-court | 37 x 34 | 1258 | 2011 |  | Public | 100% |  | 74% | 13% | 12% |
| SPRINGWEST ACADEMY | 4-court | 33 x 18 | 594 | 1990 |  | Educational (in-house) | 100% |  | 66% | 9% | 25% |
| THE GREEN SCHOOL | 4-court | 34.5 x 20 | 690 | 2006 |  | Educational (in-house) | 100% |  | 71% | 13% | 16% |

## Appendix 2: Information not included within the national run

In January 2026 the Greater London Authority contacted all local authorities with a copy of the Active Places data (shown above in Appendix 1) to identify any changes. Where a Borough has provided this information, it is recorded in the table below for information and updates were made to Active Places to inform future local FPM runs. An explanation of the model, inclusion criteria and model parameters is given in Appendix 3.

| Site Name | Changes requested |
| --- | --- |
| Bromley |  |
| Walnuts Leisure Centre | Last refurbished in 2025 |
| Camden |  |
| All information up to date |  |
| Croydon |  |
| Monks Hill Sports Centre | 40 Available hours |
| New Addington Leisure Centre | 67.5 Available hours |
| Thornton Heath Leisure Centre | 89.5 Available hours |
| Waddon Leisure Centre | 100 Available hours |
| Ealing |  |
| W3 Club (Closes) | This site has now reopened and is called Mode Club all other details remain the same - see website www.modeclub.co.uk |
| Northolt Leisure Centre | Access should read Pay and Play |
| Action Centre | Access for both Acton Centre lines should read Pay and Play |
| Hounslow |  |
| Virgin Active Chiswick Park | Website - available hours 100.5 |
| Chiswick School | Main Hall Monday - Friday - 17.00- 22.00 Weekends 09.00 - 19.00. Total Hours available 49 (Website) |
|  | Activity Hall Monday - Friday - 17.00- 22.00 Weekends 09.00 - 19.00. Total Hours available 49 (Website) |
| David Lloyd Heston | Not available to rent - members only |
| Gunnersbury Catholic School | Available to hire - yes, available for community use - yes access sports club and association - yes |
| Heston Sports Centre | Available to hire - yes, Available for community use - yes, last refurb 2025. |
| Isleworth And Syon School | Activity Hall - redecorated last year |
| Kingsley Academy | Main Hall available for community use and hire |
|  | Activity Hall - available for community use and hire |
| Springwest Academy | Monday to Thursday 17.00 to 22.00, Friday 13.00 -22.00. Saturday 08.00 - 23.30 Sunday 08.00 - 22.00. Total hours 58.30 (website) |
| Kingston upon Thames |  |
| Kingfisher Leisure Centre | Closed - planned opening in 2027 |
| Fitness4Less (New Malden) | Closed |
| Chessington Sports Centre | Management is now Commercial Management as of 01/10/2025 |
| Merton |  |
| AELTC Community Tennis Centre | New Tennis Facility |
| Richmond upon Thames |  |
| Recently Completed an FPM |  |
| Southwark |  |
| NEW Leisure Centre ( Sep 2025) Canada Water Leisure Centre: Sports Hall | Marked courts 4; area 690m2; length 34.5m; width 20m; Peak hours 44.5; available hours 101 |
| Seven Islands Leisure Centre | Permanently Closed |
| Westminster |  |
| Recently conducted an FPM report | Information is up to date |

## Appendix 3: Model Description, Inclusion Criteria and Model Parameters

Included within this Appendix are the following:

- Model Description

- Facility Inclusion Criteria

- Model Parameters

Model Description

Background

The Facilities Planning Model (FPM) is a computer-based supply/demand model, which has been developed by Edinburgh University in conjunction with sport Scotland and Sport England since the 1980s.

The model is a tool for helping to assess the strategic provision of community sports facilities in an area.  It is currently applicable for use in assessing the provision of the following facilities that Sport England holds substantial demand data for:

- sports halls

- swimming pools

- indoor bowls centres

- artificial grass pitches (AGPs)

Use of FPM

The FPM is one of Sport England’s principal tools and has been developed as a means for:

- Assessing the requirements for different types of community sports facilities on a local, regional and national scale.

- Helping local authorities to determine an adequate level of sports facility provision to meet their local needs.

- Helping to identify strategic gaps in the provision of sports facilities.

- Comparing alternative options for planned provision, taking account of changes in demand and supply.  This includes testing the impact of opening, relocating and closing facilities, and the likely impact of population changes on the need for sports facilities.

The FPM has been used in the assessment of Lottery funding bids for community facilities and as a principal planning tool to assist local authorities in planning for the provision of community sports facilities.

How the Model Works

In its simplest form, the model seeks to assess whether the capacity of existing facilities for a particular sport is capable of meeting local demand for that sport, considering how far people are prepared to travel to such a facility.

In order to do this, the model compares the capacity of facilities (supply) within an area against the demand for that facility from the local population, similar to other social gravity models.

To do this, the FPM converts both demand (in terms of people) and supply (facilities) into a single comparable unit.  This unit is ‘visits in the weekly peak period’.  Once converted, demand and supply can be compared.

The FPM uses a set of parameters to define how facilities are used and by whom.  These parameters are primarily derived from a combination of data, including actual user surveys from a range of sites across the country in areas of good supply, together with participation survey data.  These surveys provide core information on the profile of users such as their age and gender, how often they visit, the distance travelled, duration of stay, as well as information on the facilities themselves such as programming, peak times of use and capacity of facilities.

This survey information is combined with other data sources to provide a set of model parameters for each facility type.  The original core user data for halls and pools comes from the National Halls and Pools survey undertaken in 1996.  This data previously formed the basis for the National Benchmarking Service.  For AGPs, the core data used comes from the user survey of AGPs carried out in 2005/06 jointly with Sport Scotland.

User survey data from the National Benchmarking Service and other appropriate sources are used to update the model’s parameters on a regular basis.  The parameters are set out at the end of the document, and the main data sources analysed are:

Active Lives:

  - For the adult survey, this data is collected by an online survey or paper questionnaire on behalf of Sport England.  Each annual sample includes approximately 175,000 people and covers the full age/gender range.  Detailed questions are asked about over 200 separate sport categories in terms of participation and frequency.

  - For the children and young people survey, this data is collected through schools, with up to three mixed ability classes in up to three randomly chosen year groups completing an online survey.

National Benchmarking Service (now ceased):

  - This was a centre-based survey whose primary purpose was to enable centres to benchmark themselves against other centres. Sample interviews were conducted on site.  The number of people surveyed varied by year depending on how many centres took part. Approximately 10,000 swimmers and 3,500 sports hall users were surveyed per year.  This data was used for journey times, establishing proportions of particular activities in different hall types, the duration of activities and the time of activity (peak period).

Moving Communities Customer Experience Survey:

  - Annual online survey distributed by participating local authorities and operators via email.  Email invites are sent to any member or customer on their database who has attended their leisure centre in the last 3 months.  The results inform the travel mode that residents use to access facilities.

Scottish Health:

  - Annual survey of approximately 6,600 people (just under 5,000 adults).  This data is primarily used to assess participation, frequency and activity duration.

Other data is used where available.  For example, the following data sources are among those that were used to inform some of the underlying assumptions and parametres in the model and cross-check results:

  - Children’s Participation in Culture and Sport, Scottish Government, 2008

  - Young People’s Participation in Sport, Sports Council for Wales, 2009

  - Health and Social Care Information Centre, Lifestyle Statistics, 2012

  - Young People and Sport, Sport England, 2002

  - Data from Angus Council, 2013/14

  - National Pools and Halls Survey, 1996

  - This survey was used to obtain capacities per sports hall for differing sport types for programming data.

Calculating Demand

Demand is calculated by applying the user information from the parameters, as referred to above, to the population.  This produces the number of visits for that facility that will be demanded by the population.

Depending on the age and gender composition of the population, this will affect the number of visits an area will generate.  In order to reflect the different population profiles of the country, the FPM calculates demand based on the smallest census groupings.  These are output areas (OAs).

The use of OAs in the calculation of demand ensures that the FPM is able to reflect and portray differences in demand in areas at the most sensitive level based on available census information.  Each OA used is given a demand value in visits in the weekly peak period by the FPM.

Calculating Supply Capacity

A facility’s capacity varies depending on the following factors:

Its size (area of pool or hall, number of pitches or rinks)

How many hours the facility is available for use by the community in the weekly peak period

How many visits can be accommodated by the facility at any one time (pools = 1 user/6 sqm, main halls/barns = 8 users/court, activity halls/studios = 15 users/court, AGPs = 28 users/pitch, indoor bowls = 6 users/rink)

The FPM calculates the maximum number of visits that a facility can accommodate, which gives it a theoretical capacity in visits in the weekly peak period.

Based on travel time information (see Travel Time section) taken from the user surveys, the FPM then calculates how much demand would be met by the facility, having regard to its capacity and how much demand can travel to the facility.  The FPM includes an important feature of spatial interaction.  This feature takes account of the location and capacity of all the facilities, having regard to their access and the size of demand, and assesses whether the facilities are in the right place to meet the demand.

It is important to note that the FPM does not simply add up the total demand within an area and compare that to the total supply within the same area.  This approach would not take account of the spatial aspect of supply against demand in a particular area.  For example, if an area has a total demand for five facilities and there are currently six facilities within the area, it would be too simplistic to conclude that there is an over-supply of one facility as this approach would not take account of whether the five facilities are in the correct location for local people to use them within that area.  It may be that all the facilities are in one part of the local authority area, leaving other areas under-provided.  An assessment of this kind would not reflect the true picture of provision.  The FPM is able to assess supply and demand within an area based on the needs of the population within that area.

In making calculations as to supply and demand, visits made to sports facilities are not artificially restricted or calculated by reference to administrative boundaries, such as local authority areas.  Users are generally expected to use their closest facility.  The FPM reflects this through analysing the location of demand against the location of facilities, allowing for cross-boundary movement of visits.  For example, if a facility is on the boundary of a local authority, users will generally be expected to come from the population living close to the facility, but who may be in a neighbouring authority.

Calculating the Capacity of Sports Halls – Badminton Court Equivalents

The capacity of sports halls is calculated in the same way as described above, with each sports hall site having a capacity in visits in the weekly peak period.  In order for this capacity to be meaningful, these visits are converted into the equivalent of main hall badminton courts.  This ‘court’ figure is often mistakenly read as being the same as the number of ‘marked courts’ at the sports halls in the Active Places data, but it is not the same.  There will usually be a difference between this figure and the number of ‘marked courts’ in Active Places.

The reason for this is that the badminton court equivalents are of all the main and activity halls’ capacities; this is calculated based on hall size (area) and whether it is a main hall or an activity hall.  This gives a more accurate reflection of the overall capacity of the halls than simply using the ‘marked courts’ figure, for two reasons:

  - In calculating the capacity of halls, the model uses a different at-one-time parameter for main halls and for activity halls. Activity halls have a greater at-one-time capacity than main halls.  Marked courts can sometimes not properly reflect the size of the actual main hall.  For example, a hall may be marked out with four courts, when it has space for three courts.  As the model uses the ‘courts’ as a unit of size, it is important that the hall’s capacity is included as a ‘three-court unit’ rather than a ‘four-court unit’.

  - It is often difficult to visualise how much hall space there is when expressed as visits in the weekly peak period.  To make things more meaningful, this capacity is converted back into badminton court equivalents and is noted in the output tables as hall space in courts.

Facility Attractiveness – for Halls and Pools Only

Not all facilities are the same, and users will find certain facilities more attractive to use than others.  The model attempts to reflect this by introducing an attractiveness weighting factor, which affects the way visits are distributed between facilities.  Attractiveness, however, is very subjective.  Currently weightings are only used for sports hall and swimming pool modelling.

Attractiveness weightings are based on the following:

Age and refurbishment weighting (pools and halls): The older a facility is, the less attractive it will be to users.  It is recognised that this is a general assumption and that there may be examples where older facilities are more attractive than newly built ones due to excellent local management, programming and sports development.  Additionally, the date of any significant refurbishment is also included within the weighting factor; however, here attractiveness is set lower than a new build of the same year.  It is assumed that a refurbishment that is older than 20 years will have a minimal impact on the facility’s attractiveness.  The information on year built/refurbished is taken from Active Places.  A graduated curve is used to allocate the attractiveness weighting by year.  This curve levels off at 70 years old with a 20% weighting.  The refurbishment weighting is slightly lower than the new built year equivalent.

Management and ownership weighting (halls only): Due to the large number of halls being provided by the education sector, an assumption is made that these halls will generally not provide as balanced a programme as halls run by local authorities, trusts, etc, with school halls more likely to be used by teams and groups through block booking.  A less balanced programme is assumed to be less attractive to a general pay-and-play user than a standard local authority leisure centre sports hall with a wider range of activities on offer.

To reflect this, two weighting curves are used:

  - High weighted curve – includes non-education management (third party) and a more balanced programme, therefore more attractive

  - Lower weighted curve – includes educational halls managed in-house, therefore less attractive

Commercial facilities (halls and pools): While there are relatively few sports halls provided by the commercial sector, an additional weighting factor is incorporated within the model to reflect the cost element often associated with commercial facilities.  For each population OA, the Indices of Multiple Deprivation (IMD) score is used to limit whether people will use commercial facilities.  The assumption is that the higher the IMD score (less affluence), the less likely the population of the OA would choose to go to a commercial facility.

The English Indices of Deprivation 2019, produced by the Ministry of Housing, Communities and Local Government, measure relative levels of deprivation in 32,844 lower super output areas in England.  IMD is an overall relative measure of deprivation constructed by combining seven domains of deprivation according to their relative weights.

Used Capacity

Used capacity refers to how much of a facility’s theoretical capacity is being used.  At first this can appear to be unrealistically low, with local authority figures being in the 50-60% region.  Without any further explanation, it would appear that facilities are half empty.  The key point is not to see a facility’s theoretical maximum capacity (100%) as being an optimum position.  In practice this would mean that a facility would need to be completely full every hour it was open during the peak period.  This would be both unrealistic from an operational perspective and undesirable from a user’s perspective, as the facility would be completely full.

For example, a 25m four-lane pool has a theoretical capacity of 2,260 visits per week during a 52.5-hour peak period.  Usage of a pool will vary throughout the evening, with some sessions being busier than others through programming, such as an aqua aerobics session between 19:00 and 20:00 and lane swimming between 20:00 and 21:00 (see table below).  Other sessions will be quieter, such as between 21:00 and 22:00.  This pattern of use would mean a total of 143 swims taking place.  However, the pool’s maximum theoretical capacity is 264 visits throughout the evening.  In this instance the pool’s used capacity for the evening would be 54%.

| Visits per hour | 16:00-17:00 | 17:00-18:00 | 18:00-19:00 | 19:00-20:00 | 20:00-21:00 | 21:00-22:00 | Total visits for the evening |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Theoretical maximum capacity | 44 | 44 | 44 | 44 | 44 | 44 | 264 |
| Actual usage | 8 | 30 | 35 | 50 | 15 | 5 | 143 |
| % of capacity used | 18% | 68% | 80% | 114% | 34% | 11% | 54% |

Comfort Factor – Halls and Pools

If the facilities were full to their theoretical capacity, there would simply not be the space to undertake the activity comfortably.  In addition, there is a need to take account of a range of activities taking place which have different numbers of users; for example, aqua aerobics will have significantly more participants than lane swimming sessions.  Additionally, there may be times and sessions that, while being within the peak period, are less busy and so will have fewer users.

To account for these factors the notion of a ‘comfort factor’ is applied within the model.  For swimming pools, 70%, and for sports halls, 80%, of their theoretical capacity is considered as indicating where a facility starts to become uncomfortably busy.  This should be seen only as a guide to help flag when facilities are becoming busier, rather than as a ‘hard threshold’.

The comfort factor is also used to increase the capacity needed to comfortably meet unmet demand.  If this comfort factor is not applied, then any facilities provided will be operating at their maximum theoretical capacity, which is not desirable as previously noted.

The comfort factor is not applied to AGPs due to the fact they are predominantly used by teams that have a set number of players, therefore, the notion of having a ‘less busy’ pitch is not applicable.

Travel Times

The model includes three different modes of travel:

- driving

- walking and wheeling

- public transport and cycling

Car access is also considered.  In areas where there is less access to a car, the model reduces the number of visits made by car and increases those made on foot for shorter journeys and public transport for longer journeys.

Overall, surveys have shown that most visits to swimming pools, sports halls and AGPs are made by driving, with a significant minority of visits to facilities being made on foot.

| Facility | Driving | Walking | Public Transport | Cycling |
| --- | --- | --- | --- | --- |
| Swimming Pool | 74% | 18% | 4% | 4% |
| Sports Hall | 76% | 19% | 2% | 3% |
| AGP Combined | 79% | 18% | 3% | Unknown |
| AGP Football | 74% | 22% | 4% | Unknown |
| AGP Hockey | 97% | 2% | 1% | unknown |

The model includes a distance delay function where the further a user is from a facility, the less likely they will travel.  Set out below is the survey data with the percentage of visits made within each of the travel times.  This shows that almost 90% of all visits, both by car and on foot, are made within 20 minutes.  Hence, 20 minutes is often used as a rule of thumb for the coverage for sports halls and swimming pools.

| Minutes | Swimming pools: drive | Swimming pools: walk | Sports halls: drive | Sports halls: walk |
| --- | --- | --- | --- | --- |
| 0-10 | 56% | 53% | 54% | 55% |
| 11-20 | 35% | 34% | 36% | 32% |
| 21-30 | 7% | 10% | 7% | 10% |
| 31-45 | 2% | 2% | 2% | 3% |

For AGPs, there is a similar pattern to halls and pools, with hockey users observed as travelling slightly further (89% travel up to 30 minutes).  Therefore, a 20-minute travel time can also be used for combined and football, and 30 minutes for hockey.

| Minutes | AGP combined - drive | AGP combined - walk | AGP football - drive | AGP football - walk | AGP hockey - drive | AGP hockey - walk |
| --- | --- | --- | --- | --- | --- | --- |
| 0-10 | 28% | 38% | 30% | 32% | 21% | 60% |
| 10-20 | 57% | 48% | 61% | 50% | 42% | 40% |
| 20-40 | 14% | 12% | 9% | 15% | 31% | 0% |

NOTE: These are approximate figures and should only be used as a guide

Ordnance Survey’s Multi-Modal Network with average speed data is used to calculate the off-peak drive times between facilities and the population, observing any one-way and turn restrictions that apply.  These travel times have been validated against national survey work, and therefore are based on the actual travel patterns of users.

The network is also used to calculate walk times along paths and roads, excluding motorways and trunk roads.  A standard walking speed of 3 mph is used for all journeys.

Due to the complexity of public transport timetables, public transport travel times in the model are assumed to be twice as long as driving times.

A cycling speed of 12 mph is used for roads and shared use cycleways, which is approximately half the average speed of driving in urban areas.  A speed of 7.5 mph is used for tracks and off-road travel.

Facility Inclusion Criteria

Sports Halls

- The following inclusion criteria were used for this analysis:

  - Include all operational sports halls available for community use, ie, pay-and-play, membership, sports club/community association.

  - Exclude all halls not available for community use, ie, private use.

  - Exclude all halls where the main hall is less than 3 courts in size.

  - Include all ‘planned’, ‘under construction’, and ‘temporarily closed’ facilities only where all data is available for inclusion.

  - Where opening times are missing, availability is included based on similar facility types.

  - Where the year built is missing, assume date 1975.

- Facilities over the border in Wales and Scotland included, as supplied by Sport Scotland and Sport Wales.

Model Parameters

Sports Halls Parameters

| At-one-time Capacity |  | 32 users per 4-court hall 15 users per 144 square meters of activity hall |  |
| --- | --- | --- | --- |
| Coverage Maps |  | Car: 20 minutes Walking: 1.6 km Public transport: 20 minutes at about half the speed of a car NOTE: Travel times are indicative, within the context of a distance decay function of the model |  |
| Duration |  | 60 minutes |  |
| Percentage Participation Frequency per Week |  | Age; 0-15; 16-24; 25-34; 35-44; 45-59; 60-79. Male; 21.7%; 16.3%; 12.9%; 10.8%; 9.2%; 6.4%. Female; 24.7%; 16.8%; 15.1%; 13.6%; 13.5%; 10.7%. Age; 0-15; 16-24; 25-34; 35-44; 45-59; 60-79. Male; 0.60; 0.96; 0.88; 0.80; 0.94; 1.02. Female; 0.65; 1.20; 1.14; 1.01; 1.11; 0.96. |  |
| Peak Period Proportion in Peak Period |  | Weekday: 9:00 to 10:00, 17:00 to 22:00 Weekend: 08:00 to 16:00 Total: 46 hours 62% |  |

For example, it is estimated that 7.72% of 16-24 year old males will demand to use an AGP 1.67 times a week.  This calculation is done separately for the 12 age/gender groupings.

Census OAs are the smallest grouping of census population data and provide the population information on which the FPM’s demand parameters are applied.  A demand figure can then be calculated for each OA based on the population profile. There are over 178,600 OAs in England.  An OA has a target value of 125 households per OA.

Choosing a date in the mid-1970s ensures that the facility is included, while not overestimating its impact within the run.